# Theremin Gloves — Design Document (Revised)

Course: COEN 390 / ELEC 390, Concordia University, Winter 2026
Team 5: Marie Ella Cambay, Niraj Patel, Ayan Pirani, Nirthika Ilaiyarajah, Matei Moldovan

## At A Glance

| Item | Current build |
| --- | --- |
| Hardware | Two Arduino Nano 33 BLE Sense gloves over BLE |
| BLE host | BleSessionManager singleton using Nordic BLE 2.11.0 |
| Main audio path | ThereminAudioEngine -> AudioTrack |
| Background audio path | ThereminBackgroundAudioService with its own engine and 20 ms sync loop |
| Recording path | PcmListener -> RecordingManager -> file -> RecordingRepository |
| Public Play tones | 10 |
| Audio buffer | AUDIO_WRITE_FRAMES = 1024 at 48 kHz |
| Current orientation policy | portrait-locked activities |

This document is intentionally current-build specific. If an older Sprint note conflicts with this file, the current code and this audited draft take precedence.

## Architecture Diagram

flowchart LR
    A[Pitch Glove<br/>ThereminGlove<br/>Arduino Nano 33 BLE Sense]
    B[Volume Glove<br/>ThereminGloveVol<br/>Arduino Nano 33 BLE Sense]
    C[BleSessionManager]
    D[BleSnapshot]
    E[MainActivity]
    F[ThereminBackgroundAudioService]
    G[Foreground ThereminAudioEngine]
    H[Service ThereminAudioEngine]
    I[Foreground AudioTrack]
    J[Service AudioTrack]
    K[Speaker]
    L[PcmListener]
    M[RecordingManager]
    N[WAV / AAC File]
    O[RecordingRepository]
    P[LibraryActivity]

    A -->|BLE notify/write| C
    B -->|BLE notify/write| C
    C --> D
    D --> E
    D --> F
    E --> G
    F --> H
    G --> I --> K
    H --> J --> K
    G --> L --> M --> N
    H --> L
    M -.onRecordingStopped callback.-> E
    E --> O --> P

## Activity Flow Diagram

flowchart LR
    A[LaunchActivity] --> B[HomeActivity]
    A --> C[MainActivity]
    B --> D[ConnectGlovesActivity]
    B --> E[CalibrationActivity]
    D --> C
    E --> C
    C --> D
    C --> E
    C --> F[LibraryActivity]
    C --> G[SettingsActivity]
    C --> H[BeatMakerActivity]

## 1. System Overview

Theremin Gloves is an Android application that turns two BLE-connected Arduino Nano 33 BLE Sense gloves into a wireless gesture instrument. The pitch glove (ThereminGlove) streams wrist-angle telemetry that becomes frequency, the volume glove (ThereminGloveVol) streams wrist-angle telemetry that becomes amplitude, and the phone synthesizes audio locally in real time through AudioTrack. The product targets music students, hobbyist musicians, creators, and demo-oriented performers who want an expressive electronic instrument without physical contact, external synth hardware, or cloud services.

## 2. System Architecture

This section moves from screens to shared classes to the live data path so a reviewer can understand the system top-down.

### Activity classes and their roles

- LaunchActivity: launcher/loader screen. It warms heavy dependencies with AppLaunchWarmup.begin(...), initializes BLE, resolves Bluetooth permissions and Bluetooth-off state, then routes first-time users to HomeActivity and returning users to MainActivity.

- HomeActivity: setup/onboarding screen. It prompts for BLE prerequisites, triggers BleSessionManager.maybeStartAutoConnect(), shows pair status, and auto-opens Play when both gloves are connected.

- MainActivity: the Play screen. It owns the foreground ThereminAudioEngine, foreground DrumEngine, recording UI, tone/effects/scale/octave controls, Beat Maker preview overlay, and the handoff logic between foreground playback and ThereminBackgroundAudioService.

- ConnectGlovesActivity: manual BLE control screen. It exposes connect/disconnect/reconnect actions for each glove and shows per-glove connection detail from BleSnapshot.

- CalibrationActivity: calibration screen. It edits a mutable CalibrationDraft, captures neutral positions, previews live audio through ThereminBackgroundAudioService.beginCalibrationPreview(...), copies finalized values back into an AppSettings object, and then persists them through SettingsStore.

- LibraryActivity: recording browser/player. It loads metadata from RecordingRepository, supports search, folder organization, playback through MediaPlayer, rename/delete/move flows, playback modes, and custom date/duration filtering.

- SettingsActivity: settings screen. It toggles background audio, extended frequency range, pitch/volume direction, rename-dialog behavior, recording quality, calibration-guide visibility, and sensitivity response curve.

- BeatMakerActivity: dedicated 16-step sequencer editor. It uses StepGridView, PianoKeyboardView, and a local preview AudioTrack to edit drum/bass/piano patterns without going through the theremin audio engine.

### Non-Activity classes and their roles

- BleSessionManager: process-wide BLE host. It scans, connects, reconnects, runs the watchdog, stores cached glove MAC addresses, parses telemetry packets, exposes immutable BleSnapshot state, and handles Bluetooth permission/prompt flows.

- BleSnapshot: immutable read model describing Bluetooth state, per-glove connection state, latest telemetry, stale-data warnings, and recent event text for the UI.

- ThereminGloveBleManager: Nordic BleManager wrapper for one glove connection. It validates the custom service/characteristics, enables notifications, writes commands, and forwards callbacks to BleSessionManager.

- PlayMappingState: pure mapping state for Play. It converts live glove angles into mapped frequency/volume targets, clamps ranges, applies the sensitivity curve, and zeroes output when the instrument is not ready.

- ThereminAudioEngine: real-time synthesis engine. It owns the AudioTrack, synthesizes the theremin tone, applies vibrato/effects/scale snap, mixes optional DrumEngine PCM into the same mono buffer, exposes a visualizer snapshot, and provides the PcmListener tap used for recording.

- ThereminBackgroundAudioService: foreground audio service used when playback must survive outside the Play screen and during calibration preview. It owns its own ThereminAudioEngine, its own DrumEngine, and a 20 ms sync loop that mirrors BleSnapshot and saved settings into the service-owned engine.

- DrumEngine: PCM drum/bass engine. It precomputes PCM assets, schedules step timing, mixes active voices into a caller-owned mono buffer, and supports preset slots, custom Beat Maker patterns, BPM, bass, and piano synth modes.

- SequencerClock: self-rescheduling 16th-note clock used by the beat system to advance steps without cumulative drift.

- RecordingManager: ThereminAudioEngine.PcmListener implementation. It captures the mixed mono render buffer before stereo duplication, writes WAV directly, or encodes AAC through MediaCodec/MediaMuxer.

- RecordingRepository: SQLite metadata store for recordings and folders. It persists file path, name, duration, creation time, folder membership, and quality.

- RecordingExportManager: creates a user-visible export copy in Music/Theremin Gloves Recordings while the app-private original remains under internal storage.

- SettingsStore: single-row SQLite persistence for theremin settings plus shared-preference helpers for small UI flags.

- AppSettings: value object for persisted theremin/calibration/effects/sensitivity settings and their defaults.

- CalibrationDraft: mutable staging object for unsaved calibration edits, with sanitization and summary formatting.

- PlayUiText: Play-screen text formatter for titles, live frequency/volume strings, and stage-mode pills.

- AppLaunchWarmup: process-wide warmup cache for SettingsStore, AppSettings, RecordingRepository, and a warmup DrumEngine.

- RecordingListAdapter: RecyclerView.Adapter for library folders and recordings, including drag-into-folder and multi-select support.

- TopNavBarView: reusable top bar with back button, left action, glove status icons, optional overflow, and helper navigation utilities.

- BottomNavBarView: reusable five-tab bottom navigation bar for Play, Connect, Cal, Library, and Settings.

- NavigationUtils: screen navigation helpers and the reusable Poller class used by multiple Activities for periodic UI refresh.

- ThereminVisualizerView: custom waveform renderer for live Play audio.

- ToneKnobView: custom rotary selector for the public tone cycle.

- KnobControlView: custom dial control reused for calibration-style numeric adjustment.

- StepGridView: custom 13×16 drum/bass step-grid renderer and editor.

- PianoKeyboardView: on-screen piano keyboard used in Beat Maker and Play melody input.

- PianoStepStripView: piano-step strip helper used by the sequencer UI.

- InsetAwareScrollView: inset-aware scroll container that adds system-bar padding automatically.

- Instrument and SynthInstrument: lightweight PCM instrument abstractions retained for synth/sample wrappers inside the beat system.

### Data flow: Arduino glove to audio output

- Each glove’s Arduino Nano 33 BLE Sense computes orientation from its IMU and advertises a custom BLE service.

- The glove firmware sends ASCII packets such as ACTIVE_DELTA_DEG:<float>, NEUTRAL_ROLL_DEG:<float>, and DIRECTION:<text> over the TX notify characteristic.

- BleSessionManager scans for devices named ThereminGlove and ThereminGloveVol, connects through ThereminGloveBleManager, enables notifications, and parses each packet in handleNotification(...).

- Parsed glove state is copied into a fresh immutable BleSnapshot returned by BleSessionManager.getSnapshot().

- In foreground Play, MainActivity.refreshUiFast() calls play.syncLive(snapshot) and play.recompute(snapshot), then pushes the latest targets into ThereminAudioEngine with audioEngine.setTargets(...).

- In background/calibration preview, ThereminBackgroundAudioService.pushTargets(...) performs the same mapping every SYNC_TICK_MS = 20 ms and pushes targets into the service-owned engine.

- ThereminAudioEngine smooths target frequency/volume, synthesizes the current tone, optionally mixes DrumEngine, duplicates the mono buffer to stereo, and writes it to AudioTrack.

- In parallel, the same mono render buffer is forwarded through the PcmListener tap to RecordingManager, which writes WAV or AAC output.

- When recording stops, MainActivity receives the callback, optionally exports a user-visible copy, and saves recording metadata through RecordingRepository.

The practical separation is clean: BLE owns live telemetry, mapping owns control conversion, the audio engine owns synthesis, and the repository owns persisted metadata.

### Static singleton pattern in BleSessionManager

BleSessionManager is implemented as a process-wide static singleton: all mutable BLE state lives in static fields, all public entry points are static, and a static main-thread Handler MAIN serializes BLE operations. This design is used because BLE connectivity must survive screen changes cleanly. HomeActivity, ConnectGlovesActivity, CalibrationActivity, MainActivity, and LaunchActivity all need the same live connection state, recent telemetry, cached device addresses, and watchdog behavior. A process-wide singleton avoids duplicate scanners, duplicate GATT sessions, conflicting reconnect timers, and the need to rehydrate BLE state every time the user changes screens.

### How MainActivity and ThereminBackgroundAudioService handle audio ownership

The app does not currently share one literal ThereminAudioEngine instance between MainActivity and ThereminBackgroundAudioService. Instead: - MainActivity owns a foreground ThereminAudioEngine while the Play screen is visible. - ThereminBackgroundAudioService owns a separate service-side ThereminAudioEngine while background audio or calibration preview is active. - MainActivity mirrors scale/effects/drum/bass/tone state into the service through static service setters such as setActiveScale(...), setReverbEnabled(...), setToneTypeNow(...), setRecordingManager(...), startIfNeeded(...), and stopIfRunning(...). - This means the user experiences a playback handoff, but the implementation is a foreground-engine/service-engine ownership swap rather than two screens sharing one common object reference.

## 3. Hardware

### Arduino platform and sensors

- Each glove uses an Arduino Nano 33 BLE Sense.

- The relevant sensor path is the on-board IMU. The Android code assumes the glove firmware converts IMU data into a roll-derived control signal before transmission.

- The app does not recompute roll from raw accelerometer/gyroscope samples. It consumes the preprocessed telemetry emitted by the glove firmware.

### Two-glove setup

- Pitch glove BLE name: ThereminGlove

- Volume glove BLE name: ThereminGloveVol

- The pitch glove controls frequency.

- The volume glove controls amplitude.

### BLE service and characteristics

- Custom service UUID: 12345678-1234-1234-1234-1234567890ab

- TX notify characteristic UUID: 12345678-1234-1234-1234-1234567890ac

- RX write characteristic UUID: 12345678-1234-1234-1234-1234567890ad

### Glove-to-phone packet formats parsed by Android

- ACTIVE_DELTA_DEG:<float>: current roll delta used for live pitch/volume mapping.

- NEUTRAL_ROLL_DEG:<float>: current neutral baseline captured by the glove.

- DIRECTION:<text>: current direction mode reported by the glove after a direction sync/toggle.

### Phone-to-glove commands sent by Android

- H: handshake / refresh request.

- N: capture the current wrist orientation as the new neutral position.

- D: toggle the glove’s direction mode.

## 4. BLE Communication Layer

### How BleSessionManager works

- Initialization: BleSessionManager.initialize(context) stores the application context, restores cached device addresses, registers the Bluetooth-state receiver once, and starts the watchdog loop.

- Scanning: maybeStartAutoConnect() and requestConnectMissingGloves() eventually call an internal scan path that looks for the two expected BLE names. A scan timeout stops discovery after SCAN_TIMEOUT_MS.

- Connecting: once a target device is found, ThereminGloveBleManager.connectTo(...) starts a Nordic BLE connection with retry(3, 250) and a configurable timeout. onReady() sends the handshake and enables notifications.

- Reconnecting: if a glove drops unexpectedly, drop(...) clears live connection state and schedules AUTO_RECONNECT_DELAY_MS reconnect logic. Cached MAC addresses let reconnect attempts skip a full name-based scan when possible.

- Watchdog: the main-thread watchdog runs every WATCHDOG_PERIOD_MS. It refreshes truth, checks connect timeouts, checks telemetry age, pings silent gloves, and forces reconnects when the connection looks alive but no telemetry is arriving.

### Exact BLE timing constants from code

- SCAN_TIMEOUT_MS = 12_000L

- CONNECT_TIMEOUT_MS = 12_000L

- AUTO_RECONNECT_DELAY_MS = 1_500L

- PING_AFTER_MS = 3_000L

- STALE_WARNING_MS = 4_500L

- STALE_RECONNECT_MS = 20_000L

- WATCHDOG_PERIOD_MS = 1_000L

### Silent disconnect detection

The app distinguishes a true Android GATT disconnect from a silent telemetry stall: - Every received telemetry packet updates glove.lastTelemetryMs. - If telemetry age exceeds STALE_WARNING_MS = 4500 ms, the glove is marked stale and the UI shows Connected • no data. - If telemetry age exceeds PING_AFTER_MS = 3000 ms and at least 2 seconds have elapsed since the last ping, BleSessionManager sends another H handshake to prompt a reply. - If telemetry age exceeds STALE_RECONNECT_MS = 20_000 ms, the app treats the session as dead, drops the glove, and schedules reconnect. - Separate logic also aborts a connect attempt if connectAttemptStartMs exceeds CONNECT_TIMEOUT_MS = 12_000 ms.

### Android 12+ permission handling

- Android 12 and higher: BLUETOOTH_SCAN and BLUETOOTH_CONNECT

- Android 11 and lower: ACCESS_FINE_LOCATION

- The permission helpers are centralized in BleSessionManager.hasRequiredPermissions(...), requestRequiredPermissions(...), and wereAllPermissionsGranted(...).

- LaunchActivity, HomeActivity, ConnectGlovesActivity, and MainActivity all call these helpers instead of duplicating version-specific permission logic.

### Nordic BLE library version and rationale

- Dependency: no.nordicsemi.android:ble:2.11.0

- The code uses Nordic’s BleManager abstraction instead of raw Android BluetoothGatt directly.

- Why this is beneficial here: it provides queued connection/notification/write operations, cleaner state callbacks, built-in retry/timeout helpers, and a smaller surface area for Android GATT race conditions than hand-written raw BluetoothGatt code.

- Inference from the code: the project relies on Nordic features such as retry(3, 250), structured notification enabling, and centralized failure callbacks to keep the BLE layer readable and robust.

## 5. IMU Processing and Gesture Mapping

### How ACTIVE_DELTA_DEG relates to NEUTRAL_ROLL_DEG

The Android app treats ACTIVE_DELTA_DEG as already relative to the glove’s neutral position. In other words, the glove firmware captures a neutral roll angle as NEUTRAL_ROLL_DEG, then sends a live delta that is measured relative to that neutral reference. The Android side does not recompute that subtraction from raw IMU values; it simply parses the already-relative delta and maps it to sound.

### Frequency mapping in PlayMappingState

PlayMappingState.recompute(...) uses:

pitchNorm = applySensitivityCurve(
    clamp((pitchActiveDeltaDeg - pitchAngleMinDeg) /
          (pitchAngleMaxDeg - pitchAngleMinDeg), 0, 1))

freq = freqMinHz + (freqMaxHz - freqMinHz) * pitchNorm

Then, if octave shift is enabled:

freq = clamp(freq * 2^octaveShift, 20, 20000)

Additional guards: - if the pitch glove has no angle, frequency falls back to freqMinHz - if the instrument is not ready, volume is forced to zero

### Volume mapping in PlayMappingState

PlayMappingState.recompute(...) uses:

volNorm = applySensitivityCurve(
    clamp((volActiveDeltaDeg - volumeAngleMinDeg) /
          (volumeAngleMaxDeg - volumeAngleMinDeg), 0, 1))

volume = 0 + (1 - 0) * volNorm

Then: - if the volume glove has no angle, volume becomes 0 - if Bluetooth is off or one glove is missing, audioTargetVolumeLinear = 0

### Smoothing in ThereminAudioEngine

- FREQ_SMOOTHING = 0.0030f

- ATTACK_SMOOTHING = 0.0046f

- RELEASE_SMOOTHING = 0.0018f

Implementation details: - smoothFreqHz += (targetFreqHz - smoothFreqHz) * FREQ_SMOOTHING - smoothVolumeLinear += (target - smoothVolumeLinear) * attackOrReleaseFactor - Attack and release use different constants so notes fade in faster than they fade out.

### Why there is no explicit dead zone

There is no dedicated Java-side dead-zone constant in the current Play mapping path. Instead, three existing mechanisms already suppress micro-jitter: - calibration lets the user choose a meaningful angle span - the sensitivity response curve reshapes normalized motion - ThereminAudioEngine smooths both pitch and volume continuously

That means the app currently solves the “tiny hand tremor” problem with calibration plus smoothing rather than a hard dead-band clamp.

## 6. Audio Engine

### AudioTrack configuration

- SAMPLE_RATE = 48000

- CHANNEL_MASK = AudioFormat.CHANNEL_OUT_STEREO

- ENCODING = AudioFormat.ENCODING_PCM_16BIT

- AUDIO_WRITE_FRAMES = 1024

- Effective stereo write size per buffer: 1024 frames × 2 channels × 2 bytes = 4096 bytes

- MIN_STREAM_BUFFER_BYTES = 4096

The engine builds a low-latency AudioTrack with: - usage: USAGE_GAME - content type: CONTENT_TYPE_SONIFICATION - low-latency performance mode on Android O+

### Current public tone set

The current Play build exposes 10 user-selectable tones, not the older 9-waveform list that earlier documents referenced: - THEREMIN: heterodyne-inspired harmonic stack with a strong second partial for the classic vocal/cello theremin color. - AIR_PAD: soft detuned sine pad with slow beating and low spectral centroid. - CELLO: bowed-string style preset with emphasized first partials and gentle FM motion. - PAD: warm harmonic pad built from a detuned sine stack. - CHOIR: soft vocal pad with shallow phase modulation and low-order harmonics. - FLUTE: near-pure flute tone with only a tiny upper-harmonic shimmer. - CLARINET: odd-harmonic closed-pipe spectrum that stays warm and woody. - TRIANGLE: triangle-derived tone with boosted odd upper partials for extra brightness. - SAW: additive saw-style spectrum using the first six harmonics. - HELICOPTER: legacy rotor-like pulse tone whose theremin pitch controls chop rate.

Hidden legacy tone strings such as SQUARE, PULSE, ORGAN, STRING, BELL, DRUM_KIT, TRUMPET, and others are still supported internally for backward compatibility, but they are not in the public tone picker.

### Vibrato

- Rate: VIBRATO_RATE_HZ = 4.2f

- Minimum depth: MIN_VIBRATO_DEPTH = 0.0003f

- Maximum depth: MAX_VIBRATO_DEPTH = 0.0014f

- Depth scaling: vibrato depth grows with current smoothed volume:

mix = clamp((smoothVolumeLinear - 0.03) / 0.35, 0, 1)
depth = MIN_VIBRATO_DEPTH + (MAX_VIBRATO_DEPTH - MIN_VIBRATO_DEPTH) * mix

So louder playing introduces slightly deeper vibrato than very quiet playing.

### PCM tap and RecordingManager

- ThereminAudioEngine defines the PcmListener interface.

- RecordingManager implements ThereminAudioEngine.PcmListener.

- The engine calls listener.onPcmSamples(monoBuffer, monoBuffer.length) before the mono buffer is duplicated into stereo for AudioTrack.

- Because the tap is pre-stereo and post-drum-mix, recordings capture the exact synthesized output rather than microphone audio.

### Background service and 20 ms sync loop

- ThereminBackgroundAudioService owns a service-side ThereminAudioEngine and DrumEngine.

- Its worker thread sleeps for SYNC_TICK_MS = 20.

- On each tick it reloads persisted settings as needed, reads BleSnapshot, applies scale/effects/drum/bass/tone/octave state, computes current frequency/volume targets, and pushes them to the service-owned audio engine.

- This same loop is also used for calibration preview.

## 7. Recording System

### RecordingManager

- Capture source: ThereminAudioEngine.PcmListener

- Input data: mixed mono PCM from the live synth path

- File output: mono input is duplicated into stereo for saved files

- Storage guard: MIN_FREE_BYTES = 10 * 1024 * 1024

### Quality modes

- LOSSLESS: WAV, PCM 16-bit, 48 kHz, stereo, no codec

- HIGH: AAC-LC, 320 kbps, 48 kHz, stereo (.m4a)

- MEDIUM: AAC-LC, 192 kbps, 48 kHz, stereo (.m4a)

- LOW: AAC-LC, 128 kbps, 48 kHz, stereo (.m4a)

### RecordingRepository schema and file location

- Database file: recordings.db

- Tables:

  - folders(id, name, created_at_ms)

  - recordings(id, file_path, display_name, duration_ms, created_at_ms, folder_id, quality)

- App-private primary recording files: getFilesDir()/recordings/

- Optional exported user-visible copies: Music/Theremin Gloves Recordings via RecordingExportManager

- Important nuance: recordings.db stores the app-private original file path. The optional exported copy is a convenience duplicate, not a separately persisted repository record.

### LibraryActivity features

- load all saved recordings from RecordingRepository

- search by display name in real time

- rename recordings

- delete recordings

- create, rename, delete, and enter folders

- drag a recording onto a folder card to move it

- multi-select move/delete

- MediaPlayer playback

- mini-player with progress bar

- playback modes: sequential, single, loop one, loop all

- quality badge display

- custom duration filter

- custom date-range filter

## 8. Settings and Persistence

### SettingsStore

- Database name: theremin_gloves.db

- Table name: app_settings

- Storage model: one row with id = 1

### app_settings columns

- id

- pitch_angle_min_deg

- pitch_angle_max_deg

- freq_min_hz

- freq_max_hz

- volume_angle_min_deg

- volume_angle_max_deg

- pitch_direction_inverted

- volume_direction_inverted

- tone_type

- pitch_enabled

- volume_enabled

- updated_at_ms

- active_scale

- octave_shift

- reverb_enabled

- reverb_mix

- delay_enabled

- delay_feedback

- delay_mix

- distortion_enabled

- distortion_gain

- sensitivity_level

- sensitivity_curve

### Migration strategy

- SettingsStore uses ensureSchema(db) plus addColumnIfMissing(...)

- It does not rely on destructive migrations

- A process-wide schemaVerifiedForProcess flag avoids repeating PRAGMA table_info(...) and ALTER TABLE checks on every screen load

### SharedPreferences flags

- Preferences file theremin_prefs

  - bg_audio_enabled

  - extended_frequency_range_enabled

  - audio_compression

  - show_rename_dialog_on_stop

- Preferences file calibration_ui_prefs

  - calibration_guide_learned

### AppSettings fields and default values

These are the persisted theremin/calibration defaults loaded into a fresh AppSettings object and used by SettingsStore plus CalibrationDraft. They are not the narrower Play-screen reset defaults used by PlayMappingState.restoreDefaults().

| Field | Default |
| --- | --- |
| pitchAngleMinDeg | 0f |
| pitchAngleMaxDeg | 90f |
| freqMinHz | 20f |
| freqMaxHz | 2000f |
| volumeAngleMinDeg | 0f |
| volumeAngleMaxDeg | 90f |
| pitchDirectionInverted | false |
| volumeDirectionInverted | true |
| toneType | THEREMIN |
| pitchEnabled | true |
| volumeEnabled | true |
| activeScale | CHROMATIC |
| octaveShift | 0 |
| reverbEnabled | false |
| reverbMix | 0.3f |
| delayEnabled | false |
| delayFeedback | 0.35f |
| delayMix | 0.4f |
| distortionEnabled | false |
| distortionGain | 3.0f |
| sensitivityLevel | MEDIUM |
| sensitivityResponseCurve | 1.0f |

Important nuance: - the SQLite schema default for tone_type is still 'SINE' for backward compatibility, but SettingsStore.load() normalizes that legacy value to THEREMIN

Separate Play-only defaults used by PlayMappingState.restoreDefaults(): - play pitch angle range: -15f to 55f - play volume angle range: -10f to 55f - play frequency range: 880f to 2000f

## 9. Calibration System

### CalibrationDraft pattern

CalibrationActivity never writes directly into AppSettings while the user is still editing. Instead: - persisted settings are loaded into a mutable CalibrationDraft - the draft is edited through the calibration UI - CalibrationDraft.sanitize() clamps angle/frequency bounds and enforces valid min/max relationships - only when the user chooses Save does draft.saveTo(settings) copy the finalized values back into AppSettings

This avoids partial writes and makes Reload/Defaults behavior straightforward.

### beginCalibrationPreview(...) / endCalibrationPreview() flow

- CalibrationActivity.syncCalibrationPreview() builds an AppSettings snapshot representing the current unsaved draft.

- It calls ThereminBackgroundAudioService.beginCalibrationPreview(context, previewSettings).

- The service stores that preview object in calibrationPreviewSettings.

- On each 20 ms service tick, pushTargets(...) uses calibrationPreviewSettings instead of the saved settings row.

- When calibration is stopped, saved, or the screen leaves the foreground, ThereminBackgroundAudioService.endCalibrationPreview() clears the preview override.

### How neutral position is captured and stored

Neutral capture is a BLE/device-side operation: 1. The user presses btnPitchNeutral or btnVolumeNeutral. 2. CalibrationActivity.handleNeutralCapture(...) checks that the corresponding glove is connected. 3. BleSessionManager.requestCaptureNeutral(isPitch) sends the N command to the glove. 4. The glove replies with NEUTRAL_ROLL_DEG:<float>. 5. BleSessionManager.handleNotification(...) stores that value into the glove’s live state (glove.neutralRollDeg). 6. Future ACTIVE_DELTA_DEG packets are then interpreted relative to that neutral.

Important nuance: the neutral roll angle is not written into SettingsStore. The persisted calibration row stores pitch/volume/frequency ranges and tone settings; the live neutral reference itself lives in the glove/session state.

## 10. End-to-End Latency Analysis

### Fixed values from the current code

- BLE notification interval assumption: typically about 7.5–20 ms on Android BLE links

- Background-service sync loop: SYNC_TICK_MS = 20 ms

- Foreground Play refresh loop: UI_TICK_MS = 50 ms

- Audio buffer: AUDIO_WRITE_FRAMES / SAMPLE_RATE = 1024 / 48000 = 0.02133 s = 21.33 ms

### Background/calibration path latency

This is the path used by ThereminBackgroundAudioService and calibration preview:

Typical = BLE notify + average sync wait + audio buffer
        = 7.5–20 ms + ~10 ms + 21.33 ms
        = ~38.83–51.33 ms

Worst case = 20 ms + 20 ms + 21.33 ms
           = ~61.33 ms

### Foreground Play path latency

On the visible Play screen, MainActivity.refreshUiFast() runs every UI_TICK_MS = 50 ms, so the control-update cadence is currently looser than the service cadence:

Typical = BLE notify + average UI wait + audio buffer
        = 7.5–20 ms + ~25 ms + 21.33 ms
        = ~53.83–66.33 ms

Worst case = 20 ms + 50 ms + 21.33 ms
           = ~91.33 ms

### Comparison against HD-11 (< 80 ms)

- Background/calibration path: comfortably under 80 ms even in the worst case.

- Foreground Play path: typical behavior is still under 80 ms, but a strict worst-case bound based on the current UI_TICK_MS = 50 ms exceeds 80 ms.

- This is why older documents that assumed AUDIO_WRITE_SAMPLES = 2048 are now outdated: the current audio buffer is smaller (1024 frames), but the real foreground limit is now the Play-screen refresh loop, not the service loop.