# Theremin Gloves — Computer Simulation Summary

Course: COEN 390 / ELEC 390, Concordia University, Winter 2026
Team 5: Marie Ella Cambay, Niraj Patel, Ayan Pirani, Nirthika Ilaiyarajah, Matei Moldovan

## Executive Summary

- The current background-audio and calibration-preview path is comfortably under the HD-11 <80 ms target.

- The visible Play screen is still acceptable in typical use, but its strict worst case is higher because UI_TICK_MS = 50 ms.

- The public Play build now exposes a curated 10-tone selector, not the older 9-waveform set.

- The default Play mapping ranges are intentionally narrower than the persisted calibration defaults so the instrument feels more controllable in a demo setting.

## 1. BLE Latency Model

The latency chain was modelled analytically from the current implementation constants: - BLE notify interval: typically about 7.5–20 ms - service sync loop: SYNC_TICK_MS = 20 ms - audio buffer: AUDIO_WRITE_FRAMES / SAMPLE_RATE = 1024 / 48000 = 21.33 ms

For the background-service path: - typical latency is approximately 38.8–51.3 ms - worst case is approximately 61.3 ms

For the visible Play path, target updates currently depend on UI_TICK_MS = 50 ms, so: - typical latency is approximately 53.8–66.3 ms - worst case is approximately 91.3 ms

Finding: - the service path satisfies the HD-11 <80 ms criterion comfortably - typical foreground use is also acceptable - strict foreground worst case exceeds the target because the current UI refresh loop is slower than the service sync loop

Design implication: - the code no longer uses the older 2048-sample assumption seen in stale docs - the present limiting factor is not the audio buffer alone, but the foreground update cadence

## 2. Frequency Mapping Range Analysis

The mapping function in PlayMappingState is a clamped linear interpolation after angle normalization and sensitivity shaping:

pitchNorm = clamp((angle - minAngle) / (maxAngle - minAngle), 0, 1)
pitchNorm = pitchNorm ^ sensitivityResponseCurve
freq = freqMinHz + (freqMaxHz - freqMinHz) * pitchNorm

The current default Play ranges are: - pitch angle: -15° to 55° - volume angle: -10° to 55° - frequency: 880 Hz to 2000 Hz

Important distinction: - these are Play-screen mapping defaults from PlayMappingState - the persisted Calibration/Settings defaults in AppSettings remain pitch 0° to 90°, volume 0° to 90°, and frequency 20 Hz to 2000 Hz

Interpretation: - these defaults bias the live instrument toward a musically useful upper register rather than a huge raw span - the calibration screen still allows much wider range customization - extended range mode raises the ceiling to 20,000 Hz, but that mode favors experimentation over precise melodic control

Finding: - a moderate calibrated angle span remains the best compromise between reachable movement and useful melodic resolution - the current default ranges are aggressive but practical for a demo-oriented theremin experience

## 3. Waveform / Tone Harmonic Analysis

The current public build exposes 10 user-selectable tones, not the older 9-waveform set. Harmonic inspection of the synthesis code in ThereminAudioEngine.sample(...) shows: - THEREMIN: harmonic stack with strong second partial for classic vocal/cello theremin color - AIR_PAD and PAD: detuned layered partials for width and slow beating - CELLO: low-order bowed-string style harmonic emphasis - CHOIR: low-order vocal-pad harmonic structure with shallow phase modulation - FLUTE: near-sine spectrum with minimal shimmer - CLARINET: odd-harmonic closed-pipe structure - TRIANGLE: triangle-derived spectrum with reinforced odd partials - SAW: additive all-harmonic structure limited to the first few partials - HELICOPTER: pulse-like rhythmic rotor effect whose hit rate follows scaled frequency

Finding: - the code intentionally favors restrained additive or phase-modulated spectra over raw discontinuous waveforms, which reduces harshness and alias-like roughness on phone speakers - the current public tone set is curated for usability rather than for preserving every legacy tone ever implemented

## 4. Scale Quantization Accuracy

Scale lock is implemented in ThereminAudioEngine.snapToScale(...) using: - precomputed MIDI frequencies for MIDI notes 0..127 - scale-specific semitone offsets for CHROMATIC, MAJOR, MINOR, and PENTATONIC - a rebuilt in-scale frequency table when the active scale changes - nearest-note binary search over that sorted table

This approach means: - no expensive per-sample logarithm or power calculation is needed in the hot path - scale quantization remains consistent across octaves - the audio thread stays allocation-free during ordinary steady-state playback

Finding: - the scale-lock design is computationally lightweight and musically coherent - the implementation is well suited to real-time theremin control because most adjacent samples remain near the same snapped note, allowing the cache path to avoid repeated searches