# Theremin Gloves — Design Document (Revised)

Course: COEN 390 / ELEC 390, Concordia University, Winter 2026
Team 5: Niraj Patel, Matei Moldovan, Nirthika Ilaiyarajah, Ayan Pirani, Marie Ella Cambay

## At A Glance

| Item | Value |
| --- | --- |
| Hardware | Two Arduino Nano 33 BLE Sense gloves + Android phone |
| BLE library | Nordic no.nordicsemi.android:ble:2.11.0 |
| BLE host | BleSessionManager static singleton |
| Foreground audio | ThereminAudioEngine → AudioTrack (MainActivity) |
| Background audio | ThereminBackgroundAudioService (20 ms sync loop) |
| Recording tap | PcmListener → RecordingManager → WAV / AAC file |
| Public Play tones | 11 (THEREMIN through HELICOPTER) |
| Audio buffer | AUDIO_WRITE_FRAMES = 1024 frames @ 48 kHz ≈ 21.3 ms |
| Sample rate | SAMPLE_RATE = 48000 Hz |
| Orientation | Portrait-locked (all activities) |
| Java source files | 34 |
| SQLite databases | 2 (theremin_gloves.db, recordings.db) |

This document is intentionally current-build specific. If an older Sprint note conflicts with this file, the current code and this audited draft take precedence.

Design rationale and references. - The summary table is included so the document exposes the main architectural elements, responsibilities, and constraints in a form that can be reviewed quickly; that is consistent with architecture-description practice in ISO/IEC/IEEE 42010 [R1]. - The portrait-locked layout should be understood as a phone-first prototype tradeoff that keeps controls stable during performance, not as a general Android best practice; modern adaptive guidance would normally prefer broader orientation support [R18].

## Architecture Diagram

flowchart LR
    A[Right Pitch Glove<br/>ThereminGlove<br/>Arduino Nano 33 BLE Sense]
    B[Left Volume Glove<br/>ThereminGloveVol<br/>Arduino Nano 33 BLE Sense]
    C[BleSessionManager]
    D[BleSnapshot]
    E[MainActivity]
    F[ThereminBackgroundAudioService]
    G[Foreground ThereminAudioEngine]
    H[Service ThereminAudioEngine]
    I[Foreground AudioTrack]
    J[Service AudioTrack]
    K[Speaker]
    L[PcmListener]
    M[RecordingManager]
    N[WAV / AAC File]
    O[RecordingRepository]
    P[LibraryActivity]

    A -->|BLE notify/write| C
    B -->|BLE notify/write| C
    C --> D
    D --> E
    D --> F
    E --> G
    F --> H
    G --> I --> K
    H --> J --> K
    G --> L --> M --> N
    H --> L
    M -.onRecordingStopped callback.-> E
    E --> O --> P

Design rationale and references. - The architecture diagram is included because a structural view is needed to show ownership boundaries between BLE, mapping, audio, recording, and playback; ISO/IEC/IEEE 42010 explicitly treats these views as part of the architecture description itself [R1].

## Activity Flow Diagram

flowchart LR
    A[LaunchActivity] --> B[HomeActivity]
    A --> C[MainActivity]
    B --> D[ConnectGlovesActivity]
    B --> E[CalibrationActivity]
    D --> C
    E --> C
    C --> D
    C --> E
    C --> F[LibraryActivity]
    C --> G[SettingsActivity]
    C --> H[BeatMakerActivity]
    G --> I[UserManualActivity]

Design rationale and references. - The activity flow diagram is included because a navigation and state-flow view complements the structural view: one explains what the system is made of, while the other explains how users move between setup, calibration, performance, and media-management states [R1].

## Application Screen Flow

Figure 1. Screenshot-based screen-flow diagram showing the principal Android screens and the main progression from launch to setup, connection, calibration, play, beat editing, library access, settings, and the in-app user manual.

Design rationale and references. - This figure is included to show the implemented Android screens using real captured UI states rather than speculative mockups, which makes the document more traceable to the shipped prototype [R1]. - The figure is intentionally arranged as a simplified screen-flow diagram rather than a polished visual design board, because the goal of the design document is to communicate screen coverage, navigation, and feature placement clearly [R1].

## 1. System Overview

Theremin Gloves is an Android application that turns two BLE-connected Arduino Nano 33 BLE Sense gloves into a wireless gesture instrument. The right pitch glove (ThereminGlove) streams wrist-angle telemetry that becomes frequency, the left volume glove (ThereminGloveVol) streams wrist-angle telemetry that becomes amplitude, and the phone synthesizes audio locally in real time through AudioTrack. The product targets music students, hobbyist musicians, creators, and demo-oriented performers who want an expressive electronic instrument without physical contact, external synth hardware, or cloud services.

Design rationale and references. - The product is designed around local, on-device synthesis rather than cloud audio because low-latency musical interaction depends on minimizing transport uncertainty and scheduling variability; both Android audio guidance and DMI literature support this decision [R7][R8][R10][R11]. - The emphasis on students, hobbyists, and demonstration-oriented performers reflects a deliberate balance between immediate learnability and longer-term expressiveness, which is a recurring goal in digital-musical-instrument design [R10].

## 2. System Architecture

This section moves from screens to shared classes to the live data path so a reviewer can understand the system top-down.

### Activity classes and their roles

- LaunchActivity: launcher/loader screen. It warms heavy dependencies with AppLaunchWarmup.begin(...), initializes BLE, resolves Bluetooth permissions and Bluetooth-off state, then routes first-time users to HomeActivity and returning users to MainActivity.

- HomeActivity: setup/onboarding screen. It prompts for BLE prerequisites, triggers BleSessionManager.maybeStartAutoConnect(), shows pair status, and auto-opens Play when both gloves are connected.

- MainActivity: the Play screen. It owns the foreground ThereminAudioEngine, foreground DrumEngine, recording UI, tone/effects/scale/octave controls, Beat Maker preview overlay, and the handoff logic between foreground playback and ThereminBackgroundAudioService.

- ConnectGlovesActivity: manual BLE control screen. It exposes connect/disconnect/reconnect actions for each glove and shows per-glove connection detail from BleSnapshot.

- CalibrationActivity: calibration screen. It edits a mutable CalibrationDraft, captures neutral positions, previews live audio through ThereminBackgroundAudioService.beginCalibrationPreview(...), copies finalized values back into an AppSettings object, and then persists them through SettingsStore.

- LibraryActivity: recording browser/player. It loads metadata from RecordingRepository, supports search, folder organization, playback through MediaPlayer, rename/delete/move flows, playback modes, and custom date/duration filtering.

- SettingsActivity: settings screen. It toggles background audio, extended frequency range, pitch/volume direction, rename-dialog behavior, recording quality, calibration-guide visibility, and sensitivity response curve.

- BeatMakerActivity: dedicated 16-step sequencer editor. It uses StepGridView, PianoKeyboardView, and a local preview AudioTrack to edit drum/bass/piano patterns without going through the theremin audio engine.

- UserManualActivity: in-app user manual. Opened from Settings, it displays all 11 manual sections as scrollable cards with hardcoded content — no external file reads or network requests. The content mirrors docs/03_User_Manual.md.

Design rationale and references. - The Activity split keeps setup, calibration, performance, beat editing, media browsing, and settings in separate workflows so the latency-critical Play path is not entangled with non-real-time UI logic [R1][R7][R9]. - LaunchActivity absorbs warm-up cost and permission triage before the user reaches Play, which is appropriate because Android audio guidance distinguishes warm-up latency from steady-state latency [R5][R7]. - Keeping the manual inside the app preserves offline availability and keeps user help in the same operational context as the instrument itself [R1].

### Non-Activity classes and their roles

- BleSessionManager: process-wide BLE host. It scans, connects, reconnects, runs the watchdog, stores cached glove MAC addresses, parses telemetry packets, exposes immutable BleSnapshot state, and handles Bluetooth permission/prompt flows.

- BleSnapshot: immutable read model describing Bluetooth state, per-glove connection state, latest telemetry, stale-data warnings, and recent event text for the UI.

- ThereminGloveBleManager: Nordic BleManager wrapper for one glove connection. It validates the custom service/characteristics, enables notifications, writes commands, and forwards callbacks to BleSessionManager.

- PlayMappingState: pure mapping state for Play. It converts live glove angles into mapped frequency/volume targets, clamps ranges, applies the sensitivity curve, and zeroes output when the instrument is not ready.

- ThereminAudioEngine: real-time synthesis engine. It owns the AudioTrack, synthesizes the theremin tone, applies vibrato/effects/scale snap, mixes optional DrumEngine PCM into the same mono buffer, exposes a visualizer snapshot, and provides the PcmListener tap used for recording.

- ThereminBackgroundAudioService: foreground audio service used when playback must survive outside the Play screen and during calibration preview. It owns its own ThereminAudioEngine, its own DrumEngine, and a 20 ms sync loop that mirrors BleSnapshot and saved settings into the service-owned engine.

- DrumEngine: PCM drum/bass engine. It precomputes PCM assets, schedules step timing, mixes active voices into a caller-owned mono buffer, and supports preset slots, custom Beat Maker patterns, BPM, bass, and piano synth modes.

- SequencerClock: self-rescheduling 16th-note clock used by the beat system to advance steps without cumulative drift.

- RecordingManager: ThereminAudioEngine.PcmListener implementation. It captures the mixed mono render buffer before stereo duplication, writes WAV directly, or encodes AAC through MediaCodec/MediaMuxer.

- RecordingRepository: SQLite metadata store for recordings and folders. It persists file path, name, duration, creation time, folder membership, and quality.

- RecordingExportManager: creates a user-visible export copy in Music/Theremin Gloves Recordings while the app-private original remains under internal storage.

- SettingsStore: single-row SQLite persistence for theremin settings plus shared-preference helpers for small UI flags.

- AppSettings: value object for persisted theremin/calibration/effects/sensitivity settings and their defaults.

- CalibrationDraft: mutable staging object for unsaved calibration edits, with sanitization and summary formatting.

- PlayUiText: Play-screen text formatter for titles, live frequency/volume strings, and stage-mode pills.

- AppLaunchWarmup: process-wide warmup cache for SettingsStore, AppSettings, RecordingRepository, and a warmup DrumEngine.

- RecordingListAdapter: RecyclerView.Adapter for library folders and recordings, including drag-into-folder and multi-select support.

- TopNavBarView: reusable top bar with back button, left action, glove status icons, optional overflow, and helper navigation utilities.

- BottomNavBarView: reusable five-tab bottom navigation bar for Play, Connect, Cal, Library, and Settings.

- NavigationUtils: screen navigation helpers and the reusable Poller class used by multiple Activities for periodic UI refresh.

- ThereminVisualizerView: custom waveform renderer for live Play audio.

- ToneKnobView: custom rotary selector for the public tone cycle.

- KnobControlView: custom dial control reused for calibration-style numeric adjustment.

- StepGridView: custom 13×16 drum/bass step-grid renderer and editor.

- PianoKeyboardView: on-screen piano keyboard used in Beat Maker and Play melody input.

- PianoStepStripView: piano-step strip helper used by the sequencer UI.

- InsetAwareScrollView: inset-aware scroll container that adds system-bar padding automatically.

- Instrument and SynthInstrument: lightweight PCM instrument abstractions retained for synth/sample wrappers inside the beat system.

Design rationale and references. - The non-Activity classes are the real architectural units that own BLE transport, control mapping, synthesis, capture, persistence, and reusable interaction widgets; calling them out separately matches the intent of an architecture description, which is to expose system elements and their relationships rather than only screen names [R1]. - The split between BleSessionManager, PlayMappingState, ThereminAudioEngine, RecordingManager, and RecordingRepository is a separation-of-concerns decision that keeps the live interactive path understandable and testable [R1][R9].

### Data flow: Arduino glove to audio output

- Each glove’s Arduino Nano 33 BLE Sense computes orientation from its IMU and advertises a custom BLE service.

- The glove firmware sends ASCII packets such as ACTIVE_DELTA_DEG:<float>, NEUTRAL_ROLL_DEG:<float>, and DIRECTION:<text> over the TX notify characteristic.

- BleSessionManager scans for devices named ThereminGlove and ThereminGloveVol, connects through ThereminGloveBleManager, enables notifications, and parses each packet in handleNotification(...).

- Parsed glove state is copied into a fresh immutable BleSnapshot returned by BleSessionManager.getSnapshot().

- In foreground Play, MainActivity.refreshUiFast() calls play.syncLive(snapshot) and play.recompute(snapshot), then pushes the latest targets into ThereminAudioEngine with audioEngine.setTargets(...).

- In background/calibration preview, ThereminBackgroundAudioService.pushTargets(...) performs the same mapping every SYNC_TICK_MS = 20 ms and pushes targets into the service-owned engine.

- ThereminAudioEngine smooths target frequency/volume, synthesizes the current tone, optionally mixes DrumEngine, duplicates the mono buffer to stereo, and writes it to AudioTrack.

- In parallel, the same mono render buffer is forwarded through the PcmListener tap to RecordingManager, which writes WAV or AAC output.

- When recording stops, MainActivity receives the callback, optionally exports a user-visible copy, and saves recording metadata through RecordingRepository.

The practical separation is clean: BLE owns live telemetry, mapping owns control conversion, the audio engine owns synthesis, and the repository owns persisted metadata.

Design rationale and references. - The packet-to-audio path is documented explicitly because action-to-sound latency is additive across communication links, scheduling boundaries, buffers, and synthesis stages; this is a central point in musical-latency literature [R11]. - The immutable BleSnapshot boundary reduces race-condition risk between transport updates, UI reads, and audio-target updates [R8][R9]. - The PCM tap is part of the live data path because the product records rendered synth output rather than room sound or microphone input [R14].

### Static singleton pattern in BleSessionManager

BleSessionManager is implemented as a process-wide static singleton: all mutable BLE state lives in static fields, all public entry points are static, and a static main-thread Handler MAIN serializes BLE operations. This design is used because BLE connectivity must survive screen changes cleanly. HomeActivity, ConnectGlovesActivity, CalibrationActivity, MainActivity, and LaunchActivity all need the same live connection state, recent telemetry, cached device addresses, and watchdog behavior. A process-wide singleton avoids duplicate scanners, duplicate GATT sessions, conflicting reconnect timers, and the need to rehydrate BLE state every time the user changes screens.

Design rationale and references. - Android BLE background guidance makes clear that scan, connect, reconnect, and connection persistence are distinct lifecycle concerns; centralizing them in one process-wide owner is therefore easier to reason about than distributing them across screens [R4].

### How MainActivity and ThereminBackgroundAudioService handle audio ownership

The app does not currently share one literal ThereminAudioEngine instance between MainActivity and ThereminBackgroundAudioService. Instead: - MainActivity owns a foreground ThereminAudioEngine while the Play screen is visible. - ThereminBackgroundAudioService owns a separate service-side ThereminAudioEngine while background audio or calibration preview is active. - MainActivity mirrors scale/effects/drum/bass/tone state into the service through static service setters such as setActiveScale(...), setReverbEnabled(...), setToneTypeNow(...), setRecordingManager(...), startIfNeeded(...), and stopIfRunning(...). - This means the user experiences a playback handoff, but the implementation is a foreground-engine/service-engine ownership swap rather than two screens sharing one common object reference.

Design rationale and references. - Android expects background playback to be owned by a service rather than by an Activity, so the foreground/service split matches platform playback guidance [R9]. - Using separate foreground and service engine instances respects the different lifecycle owners involved and avoids trying to keep one mutable real-time engine alive across both UI and background-service lifecycles [R4][R9].

## 3. Hardware

### Arduino platform and sensors

- Each glove uses an Arduino Nano 33 BLE Sense.

- The relevant sensor path is the on-board IMU. The Android code assumes the glove firmware converts IMU data into a roll-derived control signal before transmission.

- The app does not recompute roll from raw accelerometer/gyroscope samples. It consumes the preprocessed telemetry emitted by the glove firmware.

Design rationale and references. - The Arduino Nano 33 BLE Sense is an appropriate platform because it combines BLE connectivity with onboard inertial sensing, which is exactly what the product requires for a wearable motion controller [R2]. - Preprocessing orientation on the glove side reduces Android-side complexity and lets the mobile app receive compact control-ready values rather than full raw IMU streams [R2][R12].

### Two-glove setup

- Right pitch glove BLE name: ThereminGlove

- Left volume glove BLE name: ThereminGloveVol

- The right pitch glove controls frequency.

- The left volume glove controls amplitude.

Design rationale and references. - Splitting pitch and volume across two gloves preserves two independent continuous control dimensions instead of forcing a single-hand mode switch; this is consistent with DMI guidance that gesture-to-sound relationships should remain legible to the performer [R10].

### BLE service and characteristics

- Custom service UUID: 12345678-1234-1234-1234-1234567890ab

- TX notify characteristic UUID: 12345678-1234-1234-1234-1234567890ac

- RX write characteristic UUID: 12345678-1234-1234-1234-1234567890ad

Design rationale and references. - Custom GATT services and characteristics are the normal BLE mechanism for application-specific payloads, so the use of custom UUIDs is appropriate here [R3].

### Glove-to-phone packet formats parsed by Android

- ACTIVE_DELTA_DEG:<float>: current roll delta used for live pitch/volume mapping.

- NEUTRAL_ROLL_DEG:<float>: current neutral baseline captured by the glove.

- DIRECTION:<text>: current direction mode reported by the glove after a direction sync/toggle.

Design rationale and references. - The packet format is compact and human-readable because BLE characteristics are designed for small attribute payloads, and readable payloads simplify integrated hardware/software debugging [R3].

### Phone-to-glove commands sent by Android

- H: handshake / refresh request.

- N: capture the current wrist orientation as the new neutral position.

- D: toggle the glove’s direction mode.

Design rationale and references. - The three-command surface is intentionally small: handshake, neutral capture, and direction toggle are the minimum control operations needed to support connection management and calibration without introducing unnecessary protocol complexity [R3][R12].

## 4. BLE Communication Layer

### How BleSessionManager works

- Initialization: BleSessionManager.initialize(context) stores the application context, restores cached device addresses, registers the Bluetooth-state receiver once, and starts the watchdog loop.

- Scanning: maybeStartAutoConnect() and requestConnectMissingGloves() eventually call an internal scan path that looks for the two expected BLE names. A scan timeout stops discovery after SCAN_TIMEOUT_MS.

- Connecting: once a target device is found, ThereminGloveBleManager.connectTo(...) starts a Nordic BLE connection with retry(3, 250) and a configurable timeout. onReady() sends the handshake and enables notifications.

- Reconnecting: if a glove drops unexpectedly, drop(...) clears live connection state and schedules AUTO_RECONNECT_DELAY_MS reconnect logic. Cached MAC addresses let reconnect attempts skip a full name-based scan when possible.

- Watchdog: the main-thread watchdog runs every WATCHDOG_PERIOD_MS. It refreshes truth, checks connect timeouts, checks telemetry age, pings silent gloves, and forces reconnects when the connection looks alive but no telemetry is arriving.

Design rationale and references. - The BLE layer is explicitly organized as scan, connect, monitor, and reconnect because Android’s BLE background guidance treats those as separate operational concerns [R4]. - The watchdog exists because a gestural instrument must treat stale telemetry as a product failure even if the underlying GATT connection has not formally disconnected [R4][R11].

### Exact BLE timing constants from code

- SCAN_TIMEOUT_MS = 12_000L

- CONNECT_TIMEOUT_MS = 12_000L

- AUTO_RECONNECT_DELAY_MS = 1_500L

- PING_AFTER_MS = 3_000L

- STALE_WARNING_MS = 4_500L

- STALE_RECONNECT_MS = 20_000L

- WATCHDOG_PERIOD_MS = 1_000L

Design rationale and references. - These constants are intentionally measured in hundreds or thousands of milliseconds because the transport is BLE on a mobile OS rather than a hard real-time fieldbus; they balance responsiveness against false reconnects during transient radio stalls [R3][R4].

### Silent disconnect detection

The app distinguishes a true Android GATT disconnect from a silent telemetry stall: - Every received telemetry packet updates glove.lastTelemetryMs. - If telemetry age exceeds STALE_WARNING_MS = 4500 ms, the glove is marked stale and the UI shows Connected • no data. - If telemetry age exceeds PING_AFTER_MS = 3000 ms and at least 2 seconds have elapsed since the last ping, BleSessionManager sends another H handshake to prompt a reply. - If telemetry age exceeds STALE_RECONNECT_MS = 20_000 ms, the app treats the session as dead, drops the glove, and schedules reconnect. - Separate logic also aborts a connect attempt if connectAttemptStartMs exceeds CONNECT_TIMEOUT_MS = 12_000 ms.

Design rationale and references. - This logic exists because “connected but silent” is musically equivalent to failure in a live controller; telemetry freshness matters as much as connection-state callbacks [R4][R11].

### Android 12+ permission handling

- Android 12 and higher: BLUETOOTH_SCAN and BLUETOOTH_CONNECT

- Android 11 and lower: ACCESS_FINE_LOCATION

- The permission helpers are centralized in BleSessionManager.hasRequiredPermissions(...), requestRequiredPermissions(...), and wereAllPermissionsGranted(...).

- LaunchActivity, HomeActivity, ConnectGlovesActivity, and MainActivity all call these helpers instead of duplicating version-specific permission logic.

Design rationale and references. - Centralizing the permission logic is appropriate because Android 12+ introduces BLE runtime permissions that differ from the pre-Android-12 model, and duplicating that branching logic across screens would be error-prone [R5].

### Nordic BLE library version and rationale

- Dependency: no.nordicsemi.android:ble:2.11.0

- The code uses Nordic’s BleManager abstraction instead of raw Android BluetoothGatt directly.

- Why this is beneficial here: it provides queued connection/notification/write operations, cleaner state callbacks, built-in retry/timeout helpers, and a smaller surface area for Android GATT race conditions than hand-written raw BluetoothGatt code.

- The project uses Nordic features: retry(3, 250), structured notification enabling via enableNotifications(txCharacteristic).enqueue(), and centralized failure callbacks via BleManagerCallbacks.

Design rationale and references. - Nordic’s BLE library is appropriate here because it explicitly provides queued BLE operations, retries, and GATT-utility helpers that reduce common reliability problems in manual BluetoothGatt implementations [R6].

### Why useAutoConnect(false) — custom reconnect instead of Android’s built-in

The connection builder in ThereminGloveBleManager.connectTo(...) explicitly sets .useAutoConnect(false). Android’s built-in auto-connect (BluetoothDevice.connectGatt(context, true, callback)) caches device addresses in the OS and attempts reconnection indefinitely, but it is well-documented to get stuck: cached entries go stale, GATT state machines enter unrecoverable states, and callbacks are sometimes never delivered. The result can be a phantom “Connecting…” state that only a Bluetooth power-cycle can escape.

Using useAutoConnect(false) keeps every GATT session explicit. When a glove drops, BleSessionManager.drop(...) calls close() on the old manager, allocates a fresh ThereminGloveBleManager, and starts a new explicit connection — either directly to the cached MAC address (skipping a full scan) or by re-scanning for the device name. This produces reliable, diagnosable reconnect behavior and is why the watchdog exists: it drives all reconnect decisions rather than delegating them to the OS.

Design rationale and references. - Android’s BLE guidance distinguishes direct connections from auto-connect behavior; keeping reconnect policy in app code makes timing and failure handling more deterministic for a performance-oriented device [R4].

### No BLE bonding — unauthenticated connections only

The Callbacks class inside ThereminGloveBleManager leaves onBondingRequired, onBonded, and onBondingFailed as empty no-ops. Glove connections are unauthenticated: no PIN, no passkey, no bond database entry. The custom UUIDs and BLE device names serve as implicit gates without adding pairing complexity. Users never see a system pairing dialog.

Design rationale and references. - Skipping bonding is defensible in the current product scope because the gloves do not carry identity, payment, or health data, while pairing dialogs would add setup friction to a demonstration instrument [R3].

### Firmware-side BLE: ArduinoBLE library and 32-byte characteristic limit

The glove firmware (arduino/ble.cpp) uses the Arduino ArduinoBLE library (#include <ArduinoBLE.h>), which is unrelated to the Nordic Android library. The two libraries interoperate via the standard GATT protocol — Nordic on the phone side, ArduinoBLE on the Arduino side.

Each characteristic is declared as a BLEStringCharacteristic with a maximum of 32 bytes:

static BLEStringCharacteristic gTxChar(kTxUuid, BLERead | BLENotify, 32);
static BLEStringCharacteristic gRxChar(kRxUuid, BLERead | BLEWrite,  32);

bleSendLine() truncates any outgoing string to 31 characters before writing, reserving one byte for the null terminator within the BLE string type. This is a hard protocol constraint: any packet whose ASCII representation exceeds 31 characters is silently truncated. Current packet types stay well within the limit:

| Packet | Max example | Length |
| --- | --- | --- |
| ACTIVE_DELTA_DEG:<float> | ACTIVE_DELTA_DEG:-123.45 | 25 chars |
| NEUTRAL_ROLL_DEG:<float> | NEUTRAL_ROLL_DEG:-123.45 | 25 chars |
| DIRECTION:POSITIVE | DIRECTION:POSITIVE | 19 chars |

Initial values written at firmware boot: TX characteristic gets "BOOT", RX characteristic gets "ready".

Design rationale and references. - The short ASCII protocol keeps messages well within the declared characteristic-size limit of the current firmware and avoids extra packet-framing complexity on both the Arduino and Android sides [R2][R3].

### Same UUIDs for both gloves — differentiated by name only

The right pitch glove (ThereminGlove) and left volume glove (ThereminGloveVol) advertise the identical service UUID and TX/RX characteristic UUIDs. They are distinguished solely by their BLE advertised local name. BleSessionManager opens two independent GATT sessions keyed by MAC address, so UUID collisions between the two sessions are not an issue — the Android GATT stack routes callbacks by device address, not by UUID.

Design rationale and references. - Reusing one GATT schema for both gloves keeps the firmware and Android parser symmetric while still allowing the app to assign musical role by advertised device identity [R3].

## 5. IMU Processing and Gesture Mapping

### How ACTIVE_DELTA_DEG relates to NEUTRAL_ROLL_DEG

The Android app treats ACTIVE_DELTA_DEG as already relative to the glove’s neutral position. In other words, the glove firmware captures a neutral roll angle as NEUTRAL_ROLL_DEG, then sends a live delta that is measured relative to that neutral reference. The Android side does not recompute that subtraction from raw IMU values; it simply parses the already-relative delta and maps it to sound.

Design rationale and references. - Relative-to-neutral mapping is appropriate for wearables because wrist pose, glove fit, and performer stance vary from user to user; user-conducted IMU calibration is therefore more robust than assuming one fixed absolute orientation [R12].

### Frequency mapping in PlayMappingState

PlayMappingState.recompute(...) uses:

pitchNorm = applySensitivityCurve(
    clamp((pitchActiveDeltaDeg - pitchAngleMinDeg) /
          (pitchAngleMaxDeg - pitchAngleMinDeg), 0, 1))

freq = freqMinHz + (freqMaxHz - freqMinHz) * pitchNorm

Then, if octave shift is enabled:

freq = clamp(freq * 2^octaveShift, 20, 20000)

Additional guards: - if the right pitch glove has no angle, frequency falls back to freqMinHz - if the instrument is not ready, volume is forced to zero

Design rationale and references. - Linear normalization from calibrated angle span to frequency span keeps the control law explainable and adjustable, which is important for a gesture instrument intended to be learnable by non-experts [R10][R12]. - The default 20 Hz to 2000 Hz range captures a musically useful span without forcing beginners immediately into the full audible band, while octave shift and extended-range settings preserve headroom for advanced use [R7][R10].

### Volume mapping in PlayMappingState

PlayMappingState.recompute(...) uses:

volNorm = applySensitivityCurve(
    clamp((volActiveDeltaDeg - volumeAngleMinDeg) /
          (volumeAngleMaxDeg - volumeAngleMinDeg), 0, 1))

volume = 0 + (1 - 0) * volNorm

Then: - if the left volume glove has no angle, volume becomes 0 - if Bluetooth is off or one glove is missing, audioTargetVolumeLinear = 0

Design rationale and references. - Mapping amplitude from a normalized 0–1 value is the simplest continuous-volume model, and muting when the instrument is not ready is a safer response than allowing uncontrolled output [R10][R11].

### Smoothing in ThereminAudioEngine

- FREQ_SMOOTHING = 0.0030f

- ATTACK_SMOOTHING = 0.0046f

- RELEASE_SMOOTHING = 0.0018f

Implementation details: - smoothFreqHz += (targetFreqHz - smoothFreqHz) * FREQ_SMOOTHING - smoothVolumeLinear += (target - smoothVolumeLinear) * attackOrReleaseFactor - Attack and release use different constants so notes fade in faster than they fade out.

Design rationale and references. - Smoothing is necessary because continuous sensor-driven instruments are highly sensitive to step discontinuities, zipper noise, and jitter; stability matters alongside raw latency [R10][R11]. - Separate attack and release constants preserve responsiveness while reducing abrupt dropouts caused by minor hand tremors [R11][R13].

### Why there is no explicit dead zone

There is no dedicated Java-side dead-zone constant in the current Play mapping path. Instead, three existing mechanisms already suppress micro-jitter: - calibration lets the user choose a meaningful angle span - the sensitivity response curve reshapes normalized motion - ThereminAudioEngine smooths both pitch and volume continuously

That means the app currently solves the “tiny hand tremor” problem with calibration plus smoothing rather than a hard dead-band clamp.

Design rationale and references. - Omitting a hard dead zone preserves continuity, which is important for theremin-like control where small intended motions can be musically expressive; calibration plus smoothing is therefore a better fit than a hard clamp in the current design [R12][R13].

## 6. Audio Engine

### AudioTrack configuration

- SAMPLE_RATE = 48000

- CHANNEL_MASK = AudioFormat.CHANNEL_OUT_STEREO

- ENCODING = AudioFormat.ENCODING_PCM_16BIT

- AUDIO_WRITE_FRAMES = 1024

- Effective stereo write size per buffer: 1024 frames × 2 channels × 2 bytes = 4096 bytes

- MIN_STREAM_BUFFER_BYTES = 4096

The engine builds a low-latency AudioTrack with: - usage: USAGE_GAME - content type: CONTENT_TYPE_SONIFICATION - low-latency performance mode on Android O+

Design rationale and references. - Streaming AudioTrack is appropriate because the app generates audio continuously in real time rather than playing fixed clips [R9]. - 48 kHz aligns with Android’s common native device rate for low-latency output, and the 1024-frame buffer is a compromise between lower latency and underrun risk [R7][R8][R9]. - USAGE_GAME and CONTENT_TYPE_SONIFICATION are consistent with an interactive synthesized-audio workload [R9].

### Current public tone set

The current Play build exposes 11 user-selectable tones, not the older 9-waveform list that earlier documents referenced: - THEREMIN: heterodyne-inspired harmonic stack with a strong second partial for the classic vocal/cello theremin color. - AIR_PAD: soft detuned sine pad with slow beating and low spectral centroid. - CELLO: bowed-string style preset with emphasized first partials and gentle FM motion. - PAD: warm harmonic pad built from a detuned sine stack. - CHOIR: soft vocal pad with shallow phase modulation and low-order harmonics. - FLUTE: near-pure flute tone with only a tiny upper-harmonic shimmer. - CLARINET: odd-harmonic closed-pipe spectrum that stays warm and woody. - TRIANGLE: triangle-derived tone with boosted odd upper partials for extra brightness. - SAW: additive saw-style spectrum using the first six harmonics. - SQUARE: odd-harmonic square wave; hollow, reedy quality. - HELICOPTER: rotor-like pulse tone whose theremin pitch controls chop rate (0.75–12 Hz).

Hidden legacy tone strings such as PULSE, ORGAN, STRING, BELL, DRUM_KIT, TRUMPET, and others are still supported internally for backward compatibility, but they are not in the public tone picker.

Design rationale and references. - The public tone set mixes synthesis primitives with recognizable instrument-like presets so the app remains approachable while still exposing musically distinct timbres [R10]. - CLARINET is modeled around an odd-harmonic closed-pipe spectrum, while FLUTE is intentionally closer to a near-pure tone; the SAW, SQUARE, and TRIANGLE presets follow canonical Fourier-series harmonic families [R19][R20][R21][R22][R23].

### Vibrato

- Rate: VIBRATO_RATE_HZ = 4.2f

- Minimum depth: MIN_VIBRATO_DEPTH = 0.0003f

- Maximum depth: MAX_VIBRATO_DEPTH = 0.0014f

- Depth scaling: vibrato depth grows with current smoothed volume:

mix = clamp((smoothVolumeLinear - 0.03) / 0.35, 0, 1)
depth = MIN_VIBRATO_DEPTH + (MAX_VIBRATO_DEPTH - MIN_VIBRATO_DEPTH) * mix

So louder playing introduces slightly deeper vibrato than very quiet playing.

Design rationale and references. - A vibrato rate around 4–7 Hz is consistent with music-acoustics literature, and scaling depth with louder playing keeps the effect subtle at low amplitude while allowing more expressive sustain at higher amplitude [R24].

### PCM tap and RecordingManager

- ThereminAudioEngine defines the PcmListener interface.

- RecordingManager implements ThereminAudioEngine.PcmListener.

- The engine calls listener.onPcmSamples(monoBuffer, monoBuffer.length) before the mono buffer is duplicated into stereo for AudioTrack.

- Because the tap is pre-stereo and post-drum-mix, recordings capture the exact synthesized output rather than microphone audio.

Design rationale and references. - Capturing the rendered PCM stream preserves the exact generated signal and avoids room acoustics, speaker coloration, and microphone-placement variability [R14].

### Background service and 20 ms sync loop

- ThereminBackgroundAudioService owns a service-side ThereminAudioEngine and DrumEngine.

- Its worker thread sleeps for SYNC_TICK_MS = 20.

- On each tick it reloads persisted settings as needed, reads BleSnapshot, applies scale/effects/drum/bass/tone/octave state, computes current frequency/volume targets, and pushes them to the service-owned audio engine.

- This same loop is also used for calibration preview.

Design rationale and references. - The service-side sync loop exists because background playback must remain functional outside the foreground UI lifecycle, and a 20 ms service tick is much closer to interactive musical control than the 50 ms foreground UI cadence [R9][R11][R13].

### Tone synthesis recipes

All 11 public tones are computed in ThereminAudioEngine.sample() using three building blocks:

- Additive synthesis — sum of sines at integer multiples of the running phase.

- FM / PM synthesis — carrier sin(phase + I × sin(phase × R)) where I is the modulation index and R is the ratio. Depth is often scaled by volume or motion (a slow sub-LFO derived from vibratoPhase × 0.60).

- saturate(value, gain) = tanh(value × gain) — symmetric soft-clip used as the final stage on most tones. It tames peaks across summed harmonics and adds subtle warmth without the raspy edge of hard clipping.

| Tone | Technique | Key recipe details |
| --- | --- | --- |
| THEREMIN | Additive | 5 harmonics: 1.0, 0.35, 0.18, 0.07, 0.03 — strong 2nd partial gives the classic cello/vocal warmth |
| AIR_PAD | Detuned additive | Three sines: ×1.000, ×1.002, ×0.998 — microscopic detuning produces slow beating that keeps sustained notes alive |
| CELLO | FM + additive | 1:1 FM carrier (index 0.10–0.16 modulated by motion) plus harmonics 2, 3, 4 — suggests bow pressure |
| PAD | Detuned additive | Like Air Pad with wider detuning (±0.003) and a 5th partial |
| CHOIR | PM + additive | Carrier PM’d at ×2 (index 0.10–0.16); low-order harmonics 2–4; no volume scaling |
| FLUTE | PM + additive | Very shallow PM at ×2 (index ≈ 0.10); tiny 2nd harmonic; near-pure sine character |
| CLARINET | Additive (odd only) | Harmonics 1, 3, 5, 7, 9 at 1.0, 0.36, 0.19, 0.10, 0.05 — closed-pipe odd-harmonic spectrum |
| TRIANGLE | Triangle + additive | (2/π) arcsin(sin(phase)) as triangle core, then adds boosted 3rd, 5th, 7th partials |
| SAW | Additive | Harmonics 1–6 at 0.72, 0.36, 0.23, 0.16, 0.10, 0.06 — partial roll-off approximates a sawtooth |
| SQUARE | Additive (odd only) | Harmonics 1, 3, 5, 7, 9 at 0.86, 0.28, 0.15, 0.08, 0.04 — soft square with saturate(v, 0.96) |
| HELICOPTER | Pulse chop | Theremin pitch maps to pulse chop rate 0.75–12 Hz; each pulse is a brief sine burst |

Design rationale and references. - The synthesis recipes are documented because tone design is part of the product identity, not merely an implementation detail. - The use of additive, FM/PM, and soft-clipping stages provides recognizable timbral families at modest CPU cost on a phone, while canonical waveform presets remain grounded in established acoustic or Fourier models [R19][R20][R21][R22][R23].

### DrumEngine — sound synthesis

All 14 sounds are prepared at construction into float[] PCM arrays at 48 kHz. The bank is hybrid: kick/snare/hi-hats/clap and the four bass notes are loaded from bundled raw resources, while crash/toms/rim/shaker and the piano voices are synthesized in code. SoundPool is not used; all voices are normalized into the same direct-mix PCM pipeline. Rendering and sample-load jobs run in parallel across up to 4 threads during construction.

| Sound | Duration | Technique |
| --- | --- | --- |
| Kick | 400 ms | Pitch-swept harmonic sine. Frequency starts at 255 Hz, decays to ~66 Hz via exp(−30t). Three harmonics (0.70, 0.22, 0.08). Amplitude envelope exp(−6t) |
| Snare | 200 ms | 220 Hz sine body exp(−18t) mixed with HPF white noise (1-pole coeff 0.93) at exp(−14t). Body : noise ratio ≈ 0.50 : 0.52 |
| Closed hi-hat | 70 ms | HPF white noise (coeff 0.97), fast decay exp(−40t) — tight metallic click |
| Open hi-hat | 250 ms | HPF white noise (coeff 0.95), slower decay exp(−8t) — washy sustain |
| Crash cymbal | 650 ms | HPF noise (coeff 0.84) plus weak 210 Hz body sine, long decay exp(−4.5t) |
| Clap | 140 ms | Three staggered noise bursts at t = 0, 5 ms, 10 ms, each at exp(−38t). Peak-normalized to 0.88 |
| Bass E2 / A2 / D3 / G2 | 550 ms | 4 harmonics (0.55, 0.35, 0.14, 0.05). Natural pitch sag: starts 1.5% flat, rises to nominal via exp(−15t). Sustained decay exp(−3t). 2nd/3rd harmonics boosted for audibility on phone speakers |
| Hi tom | 420 ms | Pitch-swept sine from 300 Hz, exp(−t/0.14) amplitude, pitch decays via exp(−8t) |
| Low tom | 540 ms | Same as hi tom but from 180 Hz with longer 0.18 s decay constant |
| Rimshot | 50 ms | Simple 1-pole HPF Gaussian noise (coeff 1.0), very fast decay exp(−60t) |
| Shaker | 80 ms | HPF Gaussian noise (coeff 0.85), decay exp(−30t) |

The voice pool is lock-free: voiceSound and voicePos are AtomicIntegerArray with 32 slots. triggerVoice() finds the first free slot (pos < 0). mixInto() advances each active voice sample-by-sample and marks it free when it reaches the end.

Design rationale and references. - Precomputing PCM assets and mixing them directly into the engine buffer avoids expensive work on the real-time playback path, which is consistent with Android low-latency audio guidance [R8]. - The hybrid bank improves sonic variety without requiring a fully sample-based instrument library [R8][R10].

### DrumEngine — 8 preset beat patterns

Each pattern is a 6-row × 16-column boolean grid at 16th-note resolution (rows: kick, snare, closed hi-hat, open hi-hat, crash, clap). Each pattern has a paired bass pattern selecting among E2, A2, D3, and G2.

| # | Name | Character |
| --- | --- | --- |
| 0 | Rock | Kick on beats 1 & 3 with pickup; 8th-note closed hat; open hat on &-of-2, &-of-4 |
| 1 | Funk | Syncopated kick; 16th-note hat throughout; ghost + main snare on 4 |
| 2 | EDM | Four-on-floor kick; open hat on all upbeats; clap on 2 & 4; crash on beat 1 |
| 3 | Hip-Hop | Heavy syncopated kick; sparse snare on 2 & 4; 8th-note hat |
| 4 | Reggae (One-Drop) | Kick on beat 3 only; open hat on & of every beat |
| 5 | Jazz | Sparse kick comp; snare on &-of-2 and &-of-4; open hat swings on &-of-every-beat |
| 6 | Trap | Four-on-floor kick; dense 16th-note hat; clap on 2 & 4 (no snare row) |
| 7 | Latin/Samba | Clave-inspired kick; 16th-note hat; crash on beat 1; open hat on &-of-2, &-of-4 |

SequencerClock uses a self-rescheduling single-shot approach: each tick schedules the next tick based on System.currentTimeMillis() delta, so BPM changes take effect on the next step with no restart and no audible glitch.

Design rationale and references. - Preset beat patterns support quick learnability, while 16-step sequencing matches common loop-based rhythm interaction and is easy to visualize for users [R10].

### DrumEngine — piano synthesis modes

The piano covers 25 chromatic notes, C3 (MIDI 48) through C5 (MIDI 72), pre-rendered at all three modes at construction. Maximum 8 simultaneous piano voices; if all busy, the furthest-advanced voice is stolen.

| Mode | Attack | Decay shape | Technique | Character |
| --- | --- | --- | --- | --- |
| KEYS | 3 ms linear | exp(−4.5t), 800 ms total | 4 harmonics (0.70, 0.20, 0.07, 0.03) | Warm mallet/marimba |
| BELLS | 2.5 ms linear | Dual-exp: 0.72×exp(−3.8t) + 0.28×exp(−8t), 1250 ms | 4 inharmonic partials at ×1, ×2.76, ×5.43, ×8.21 | Bell/metallophone shimmer |
| ORGAN | 10 ms linear | Near-sustained: 0.82 + 0.18×exp(−2.2t), 950 ms | 4 harmonics (0.58, 0.26, 0.11, 0.05) + 5.2 Hz vibrato | Hammond drawbar character |

Design rationale and references. - The piano modes widen the melodic palette without requiring a separate full keyboard engine, while fixed note range and voice limits keep CPU and memory use bounded on a phone [R8][R10].

### DrumEngine — arpeggio patterns

When melody mode is active, the sequencer fires arpeggiated piano chords on scheduled steps. Up to 4 simultaneous root MIDI notes can be armed; each root fires the same arpeggio pattern independently, producing harmonic chords.

| Index | Name | Semitone intervals from root |
| --- | --- | --- |
| 1 | Major up | 0, 4, 7, 12 |
| 2 | Minor up | 0, 3, 7, 12 |
| 3 | Pentatonic run | 0, 2, 4, 7, 9, 12 |
| 4 | Major up & back | 0, 4, 7, 12, 7, 4 |

The arpeggio clock is sample-accurate: arpNextFireSample advances against arpSampleClock (incremented per PCM sample in mixInto()) rather than a wall-clock timer, so arpeggio timing stays locked to the audio buffer regardless of scheduling jitter.

Design rationale and references. - Driving arpeggios from the audio sample clock rather than a wall-clock timer reduces timing jitter, which is important for perceived rhythmic quality in musical systems [R11].

## 7. Recording System

### RecordingManager

- Capture source: ThereminAudioEngine.PcmListener

- Input data: mixed mono PCM from the live synth path

- File output: mono input is duplicated into stereo for saved files

- Storage guard: MIN_FREE_BYTES = 10 * 1024 * 1024

Design rationale and references. - The recording path is built around the render buffer because it guarantees signal fidelity and avoids the feedback and ambient-noise problems of microphone capture [R14].

### Quality modes

- LOSSLESS: WAV, PCM 16-bit, 48 kHz, stereo, no codec

- HIGH: AAC-LC, 320 kbps, 48 kHz, stereo (.m4a)

- MEDIUM: AAC-LC, 192 kbps, 48 kHz, stereo (.m4a)

- LOW: AAC-LC, 128 kbps, 48 kHz, stereo (.m4a)

Design rationale and references. - Offering both WAV and AAC-LC gives users an explicit quality-versus-storage tradeoff using Android-supported output formats [R14].

### RecordingRepository schema and file location

- Database file: recordings.db

- Tables:

  - folders(id, name, created_at_ms)

  - recordings(id, file_path, display_name, duration_ms, created_at_ms, folder_id, quality)

- App-private primary recording files: getFilesDir()/recordings/

- Optional exported user-visible copies: Music/Theremin Gloves Recordings via RecordingExportManager

- Important nuance: recordings.db stores the app-private original file path. The optional exported copy is a convenience duplicate, not a separately persisted repository record.

Design rationale and references. - Keeping a local metadata database plus app-private canonical files is appropriate for an offline-first media feature, while optional export preserves user access without making the public copy the source of truth [R14][R15].

### LibraryActivity features

- load all saved recordings from RecordingRepository

- search by display name in real time

- rename recordings

- delete recordings

- create, rename, delete, and enter folders

- drag a recording onto a folder card to move it

- multi-select move/delete

- MediaPlayer playback

- mini-player with progress bar

- playback modes: sequential, single, loop one, loop all

- quality badge display

- custom duration filter

- custom date-range filter

Design rationale and references. - The Library is designed as a real media-management screen rather than a temporary playback list because once capture is supported, retrieval and organization become part of the product itself [R1]. - MediaPlayer is sufficient for file playback on this screen because the Library path is not the low-latency synthesis path [R9][R25].

## 8. Settings and Persistence

### SettingsStore

- Database name: theremin_gloves.db

- Table name: app_settings

- Storage model: one row with id = 1

Design rationale and references. - A single-row settings table is appropriate because the app has one active instrument configuration per install rather than a multi-user or multi-profile model [R15].

### app_settings columns

- id

- pitch_angle_min_deg

- pitch_angle_max_deg

- freq_min_hz

- freq_max_hz

- volume_angle_min_deg

- volume_angle_max_deg

- pitch_direction_inverted

- volume_direction_inverted

- tone_type

- pitch_enabled

- volume_enabled

- updated_at_ms

- active_scale

- octave_shift

- reverb_enabled

- reverb_mix

- delay_enabled

- delay_feedback

- delay_mix

- distortion_enabled

- distortion_gain

- sensitivity_level

- sensitivity_curve

Design rationale and references. - Grouping theremin, effects, calibration, and scale settings in one row keeps the values that are loaded together at startup and playback handoff physically together in persistence [R15].

### Migration strategy

- SettingsStore uses ensureSchema(db) plus addColumnIfMissing(...)

- It does not rely on destructive migrations

- A process-wide schemaVerifiedForProcess flag avoids repeating PRAGMA table_info(...) and ALTER TABLE checks on every screen load

Design rationale and references. - The additive migration strategy is defensible because SQLite supports inexpensive ADD COLUMN schema evolution for small stable tables without requiring destructive rewrites [R16].

### SharedPreferences flags

- Preferences file theremin_prefs

  - bg_audio_enabled

  - extended_frequency_range_enabled

  - audio_compression

  - show_rename_dialog_on_stop

- Preferences file calibration_ui_prefs

  - calibration_guide_learned

Design rationale and references. - SharedPreferences is appropriate for a small number of lightweight UI flags, while the larger structured configuration stays in SQLite [R17].

### AppSettings fields and default values

These are the persisted theremin/calibration defaults loaded into a fresh AppSettings object and used by SettingsStore plus CalibrationDraft. They are not the narrower Play-screen reset defaults used by PlayMappingState.restoreDefaults().

| Field | Default |
| --- | --- |
| pitchAngleMinDeg | 0f |
| pitchAngleMaxDeg | 90f |
| freqMinHz | 20f |
| freqMaxHz | 2000f |
| volumeAngleMinDeg | 0f |
| volumeAngleMaxDeg | 90f |
| pitchDirectionInverted | false |
| volumeDirectionInverted | true |
| toneType | THEREMIN |
| pitchEnabled | true |
| volumeEnabled | true |
| activeScale | CHROMATIC |
| octaveShift | 0 |
| reverbEnabled | false |
| reverbMix | 0.3f |
| delayEnabled | false |
| delayFeedback | 0.35f |
| delayMix | 0.4f |
| distortionEnabled | false |
| distortionGain | 3.0f |
| sensitivityLevel | MEDIUM |
| sensitivityResponseCurve | 1.0f |

Important nuance: - the SQLite schema default for tone_type is still 'SINE' for backward compatibility, but SettingsStore.load() normalizes that legacy value to THEREMIN

Separate Play-only defaults used by PlayMappingState.restoreDefaults(): - pitch angle range: 0f to 90f - volume angle range: 0f to 90f - frequency range: 20f to 2 000f by default; the Settings extended-range toggle raises the usable ceiling to 20 000f

Design rationale and references. - The defaults are conservative and beginner-friendly, while the extended-range option preserves headroom for advanced experimentation [R10][R12].

## 9. Calibration System

### CalibrationDraft pattern

CalibrationActivity never writes directly into AppSettings while the user is still editing. Instead: - persisted settings are loaded into a mutable CalibrationDraft - the draft is edited through the calibration UI - CalibrationDraft.sanitize() clamps angle/frequency bounds and enforces valid min/max relationships - only when the user chooses Save does draft.saveTo(settings) copy the finalized values back into AppSettings

This avoids partial writes and makes Reload/Defaults behavior straightforward.

Design rationale and references. - The draft pattern separates editing state from committed state, which is good practice for user calibration because partial or mistaken values should remain reversible until explicit save [R12].

### beginCalibrationPreview(...) / endCalibrationPreview() flow

- CalibrationActivity.syncCalibrationPreview() builds an AppSettings snapshot representing the current unsaved draft.

- It calls ThereminBackgroundAudioService.beginCalibrationPreview(context, previewSettings).

- The service stores that preview object in calibrationPreviewSettings.

- On each 20 ms service tick, pushTargets(...) uses calibrationPreviewSettings instead of the saved settings row.

- When calibration is stopped, saved, or the screen leaves the foreground, ThereminBackgroundAudioService.endCalibrationPreview() clears the preview override.

Design rationale and references. - Preview-before-save is important because calibration quality is perceptual: the user needs to hear and feel the mapping before committing it [R12].

### How neutral position is captured and stored

Neutral capture is a BLE/device-side operation: 1. The user presses btnPitchNeutral or btnVolumeNeutral. 2. CalibrationActivity.handleNeutralCapture(...) checks that the corresponding glove is connected. 3. BleSessionManager.requestCaptureNeutral(isPitch) sends the N command to the glove. 4. The glove replies with NEUTRAL_ROLL_DEG:<float>. 5. BleSessionManager.handleNotification(...) stores that value into the glove’s live state (glove.neutralRollDeg). 6. Future ACTIVE_DELTA_DEG packets are then interpreted relative to that neutral.

Important nuance: the neutral roll angle is not written into SettingsStore. The persisted calibration row stores pitch/volume/frequency ranges and tone settings; the live neutral reference itself lives in the glove/session state.

Design rationale and references. - Keeping neutral capture on the glove side is appropriate because the glove owns the IMU frame of reference; the Android side stores mapping policy, while the live neutral baseline remains device/session state [R2][R12].

## 10. End-to-End Latency Analysis

### Fixed values from the current code

- BLE notification interval assumption: typically about 7.5–20 ms on Android BLE links

- Background-service sync loop: SYNC_TICK_MS = 20 ms

- Foreground Play refresh loop: UI_TICK_MS = 50 ms

- Audio buffer: AUDIO_WRITE_FRAMES / SAMPLE_RATE = 1024 / 48000 = 0.02133 s = 21.33 ms

Design rationale and references. - These values are listed explicitly because end-to-end musical latency is the sum of transport, scheduling, and output-buffer stages rather than only the synthesis cost [R11].

### Background/calibration path latency

This is the path used by ThereminBackgroundAudioService and calibration preview:

Typical = BLE notify + average sync wait + audio buffer
        = 7.5–20 ms + ~10 ms + 21.33 ms
        = ~38.83–51.33 ms

Worst case = 20 ms + 20 ms + 21.33 ms
           = ~61.33 ms

Design rationale and references. - The background/calibration path is stronger because the 20 ms service cadence is materially closer to interactive-audio guidance than the 50 ms UI cadence used on the foreground Play screen [R7][R8][R11].

### Foreground Play path latency

On the visible Play screen, MainActivity.refreshUiFast() runs every UI_TICK_MS = 50 ms, so the control-update cadence is currently looser than the service cadence:

Typical = BLE notify + average UI wait + audio buffer
        = 7.5–20 ms + ~25 ms + 21.33 ms
        = ~53.83–66.33 ms

Worst case = 20 ms + 50 ms + 21.33 ms
           = ~91.33 ms

Design rationale and references. - The current foreground path is limited more by UI refresh cadence than by the audio buffer itself, which identifies the next optimization target clearly [R7][R11].

### Comparison against the current project ceiling (< 80 ms)

- The 80 ms figure should be treated as a practical prototype ceiling, not as the perceptual ideal for musical interaction.

- Background/calibration path: comfortably under 80 ms even in the worst case.

- Foreground Play path: typical behavior is still under 80 ms, but a strict worst-case bound based on the current UI_TICK_MS = 50 ms exceeds 80 ms.

- This is why older documents that assumed AUDIO_WRITE_SAMPLES = 2048 are now outdated: the current audio buffer is smaller (1024 frames), but the real foreground limit is now the Play-screen refresh loop, not the service loop.

Design rationale and references. - Research does not support calling 80 ms an ideal target. A better framing is: <= 10–20 ms as the musician-facing ideal, 20–30 ms JND as a looser tolerance for some continuous theremin-like control, and < 80 ms as a practical prototype ceiling for the current project [R7][R10][R11][R13].

## 11. Expanded System Architecture Diagram

The diagram below augments Section 2 with explicit packet format labels, method names, and thread annotations.

flowchart TD
    subgraph Hardware["Hardware Layer"]
        PG["Right Pitch Glove\nArduino Nano 33 BLE Sense\nBLE name: ThereminGlove\nLSM9DS1 IMU → wrist roll angle"]
        VG["Left Volume Glove\nArduino Nano 33 BLE Sense\nBLE name: ThereminGloveVol\nLSM9DS1 IMU → wrist roll angle"]
    end

    subgraph BLE["BLE Transport Layer (Nordic BLE 2.11.0)"]
        BSM["BleSessionManager\nStatic singleton\nWatchdog: 1 000 ms\nMAC cache for fast reconnect"]
    end

    subgraph Snapshot["State Snapshot Layer"]
        BS["BleSnapshot\nImmutable value object\npitchActiveDeltaDeg\nvolumeActiveDeltaDeg\nisPitchConnected / isVolumeConnected"]
    end

    subgraph FG["Foreground Play Path (MainActivity, UI_TICK_MS=50ms)"]
        MA["MainActivity\nUI Handler 50 ms\nrefreshUiFast()"]
        PMS_FG["PlayMappingState\nnormalizeClamped()\napplySensitivityCurve()\noctave shift"]
        TAE_FG["Foreground ThereminAudioEngine\nAudioThread THREAD_PRIORITY_AUDIO\nfillBuffer() 1024 frames / 21.3ms"]
        AT_FG["Foreground AudioTrack\nUSAGE_GAME\nCONTENT_TYPE_SONIFICATION\nPERF_MODE_LOW_LATENCY"]
    end

    subgraph SVC["Background Service Path (ThereminBackgroundAudioService, SYNC_TICK_MS=20ms)"]
        TBAS["ThereminBackgroundAudioService\nForeground service (mediaPlayback)\nSync thread 20 ms\nSettings reload every 500 ms"]
        PMS_SVC["PlayMappingState\n(service-owned instance)"]
        TAE_SVC["Service ThereminAudioEngine\nSame audio thread model"]
        AT_SVC["Service AudioTrack"]
    end

    subgraph Drums["DrumEngine (mixed into both paths)"]
        DE_FG["Foreground DrumEngine\nmixInto() per 1024-frame buffer\n14 sounds + 75 piano samples\n8 preset patterns"]
        DE_SVC["Service DrumEngine"]
    end

    subgraph Recording["Recording Path"]
        RM["RecordingManager\nPcmListener.onPcmSamples()\nReentrantLock.tryLock() (non-blocking)"]
        WAV["WAV file\n48kHz stereo PCM\nRandomAccessFile"]
        AAC["AAC-LC .m4a\nMediaCodec + MediaMuxer\n320k/192k/128k"]
        RR["RecordingRepository\nrecordings.db\nfolders + recordings tables"]
        LA["LibraryActivity"]
        REM["RecordingExportManager\nMusic/Theremin Gloves Recordings"]
    end

    subgraph Persist["Persistence Layer"]
        SS["SettingsStore\ntheremin_gloves.db\napp_settings single row\n24 columns, additive migrations"]
        SP["SharedPreferences\ntheremin_prefs + calibration_ui_prefs"]
    end

    PG -->|"ACTIVE_DELTA_DEG / NEUTRAL_ROLL_DEG\nDIRECTION (notify char ...90ac)"| BSM
    VG -->|"ACTIVE_DELTA_DEG / NEUTRAL_ROLL_DEG\nDIRECTION (notify char ...90ac)"| BSM
    BSM -->|"H / N / D (write char ...90ad)"| PG
    BSM -->|"H / N / D (write char ...90ad)"| VG
    BSM --> BS

    BS --> MA
    MA --> PMS_FG
    PMS_FG --> TAE_FG
    DE_FG -->|"mixInto(monoBuffer)"| TAE_FG
    TAE_FG --> AT_FG --> Speaker

    BS --> TBAS
    TBAS --> PMS_SVC
    PMS_SVC --> TAE_SVC
    DE_SVC -->|"mixInto(monoBuffer)"| TAE_SVC
    TAE_SVC --> AT_SVC --> Speaker

    TAE_FG -->|"PcmListener.onPcmSamples()"| RM
    RM --> WAV
    RM --> AAC
    WAV --> RR
    AAC --> RR
    RR --> LA
    RM -.->|"optional export"| REM

    SS --> PMS_FG
    SS --> TBAS
    SP --> MA
    SP --> TBAS

Design rationale and references. - The expanded architecture diagram exists because a high-level box diagram is not enough for review: timing loops, ownership boundaries, and packet directions are architecturally important in a real-time mobile instrument [R1][R11].

## 12. BLE State Machine Diagram

Each glove instance in BleSessionManager follows this state machine. The watchdog (running every WATCHDOG_PERIOD_MS = 1 000 ms) drives all timed transitions.

stateDiagram-v2
    [*] --> IDLE: initialize()

    IDLE --> SCANNING: maybeStartAutoConnect()\nor requestConnectMissingGloves()

    SCANNING --> IDLE: SCAN_TIMEOUT_MS (12s) expires\nno device found
    SCANNING --> CONNECTING: BLE scan callback\nmatches ThereminGlove or ThereminGloveVol\nname

    CONNECTING --> IDLE: CONNECT_TIMEOUT_MS (12s) exceeds\nconnectAttemptStartMs
    CONNECTING --> READY: Nordic BleManager onReady()\nGATT validated, notifications enabled\nH command sent

    READY --> ACTIVE: first ACTIVE_DELTA_DEG\ntelemetry packet received

    ACTIVE --> PING: telemetry age > PING_AFTER_MS (3s)\nH command sent to glove
    PING --> ACTIVE: reply packet arrives\nlastTelemetryMs updated

    ACTIVE --> STALE_WARNING: telemetry age > STALE_WARNING_MS (4.5s)\nUI shows Connected • no data
    STALE_WARNING --> ACTIVE: new ACTIVE_DELTA_DEG\npacket arrives

    ACTIVE --> RECONNECTING: GATT onDeviceDisconnected()\ncallback received
    STALE_WARNING --> RECONNECTING: telemetry age > STALE_RECONNECT_MS (20s)\nwatchdog forces drop()
    CONNECTING --> RECONNECTING: user forces disconnect\nor BLE error

    RECONNECTING --> SCANNING: AUTO_RECONNECT_DELAY_MS (1.5s)\nelapses — no cached MAC
    RECONNECTING --> CONNECTING: AUTO_RECONNECT_DELAY_MS (1.5s)\nelapses — cached MAC available\nskips scan, connects directly

Notes: - Both the right pitch glove and left volume glove have independent state machine instances. - BleSnapshot.isPitchConnected() returns true only in the READY or ACTIVE states. - MAC addresses cached from the READY state enable the RECONNECTING→CONNECTING fast path (avoids full 12 s scan).

Design rationale and references. - A state machine is included because BLE behavior is temporal and failure-driven; prose alone is weak for communicating reconnect logic and stale-session handling [R1][R4].

## 13. Thread Model

The app uses four distinct threads. Understanding their priorities and interactions is critical for understanding audio latency and thread safety.

| Thread | Android Priority | Typical Period | Responsibilities |
| --- | --- | --- | --- |
| Main Thread | Default (0) | Event-driven | Activity lifecycle callbacks, BleSessionManager watchdog (1 s), RecordingManager.start/stopRecording(), ThereminBackgroundAudioService static setters (setToneTypeNow, setReverbEnabled, etc.) |
| UI Tick Handler | Default (0) | 50 ms | MainActivity.refreshUiFast(), BleSnapshot.getSnapshot(), PlayMappingState.syncLive()+recompute(), ThereminAudioEngine.setTargets(), ThereminVisualizerView.invalidate() |
| Service Sync Thread | THREAD_PRIORITY_AUDIO (−16) | 20 ms | ThereminBackgroundAudioService sync loop: SettingsStore.load() every 500 ms, BleSnapshot.getSnapshot(), PlayMappingState.recompute(), ThereminAudioEngine.setTargets() (service engine) |
| Audio Thread | THREAD_PRIORITY_AUDIO (−16) | ~21.3 ms | ThereminAudioEngine.fillBuffer() (1024 samples), DrumEngine.mixInto(), PcmListener.onPcmSamples() (RecordingManager tap), copyMonoToStereo(), AudioTrack.write() |

Thread safety summary:

| Field / Class | Mechanism | Rationale |
| --- | --- | --- |
| ThereminAudioEngine.targetFreqHz | volatile | Written from Main/Service thread; read in Audio thread. Single-word write is atomic on JVM. |
| ThereminAudioEngine.toneType | volatile String | Reference replacement is atomic on JVM. |
| All effect toggles in ThereminAudioEngine | volatile boolean | Single-word writes are atomic. |
| Inner loop constants in fillBuffer() | Hoisted to final locals | Prevents repeated volatile memory barriers inside 1024-sample loop. |
| DrumEngine voice pool | AtomicIntegerArray | Array element CAS operations are needed because multiple indices can be written simultaneously. A single volatile is not sufficient for array elements. |
| RecordingManager codec state | ReentrantLock.tryLock() | Non-blocking — audio thread drops buffer rather than stalling if finalization is in progress. |
| BleSnapshot | Immutable value object | Created atomically from static fields; no lock needed on the read side. |

Design rationale and references. - The thread model is documented because real-time audio, BLE UI refresh, and background playback are concurrency-sensitive subsystems; non-blocking behavior on the audio path is a first-order design requirement, not an implementation footnote [R8][R9][R11].

## 14. Audio Signal Flow — Per-Buffer Pipeline

The following ASCII flow traces the exact execution sequence inside ThereminAudioEngine.fillBuffer() for one 1024-frame buffer.

┌─────────────────────────────────────────────────────────────────────┐
│  START OF fillBuffer(short[] monoBuffer)                            │
│                                                                     │
│  1. Hoist volatile reads to final locals:                           │
│     final float  tgtFreq   = targetFreqHz      (volatile)           │
│     final float  tgtVol    = targetVolumeLinear (volatile)          │
│     final String tone      = toneType          (volatile)           │
│     final String scale     = activeScale       (volatile)           │
│     final boolean reverb   = reverbEnabled     (volatile)           │
│     final boolean delay    = delayEnabled      (volatile)           │
│     final boolean dist     = distortionEnabled (volatile)           │
│     ... (all effect params)                                         │
│                                                                     │
│  2. For i = 0 to 1023 (inner sample loop):                          │
│                                                                     │
│     a. updateFrequency():                                           │
│        smoothFreqHz += (tgtFreq - smoothFreqHz) * FREQ_SMOOTHING    │
│        vibratoPhase += TWO_PI * 4.2 / 48000                        │
│        vibratoDepth = lerp(0.0003, 0.0014, volumeMix)               │
│        freq = smoothFreqHz * (1 + sin(vibratoPhase) * depth)        │
│                                                                     │
│     b. snapToScale(freq, scale):                                    │
│        if freq == snapCacheIn  →  return snapCacheOut  (fast path)  │
│        else  →  binary search prebuilt MIDI freq table (7 iters)    │
│                 cache result in snapCacheIn / snapCacheOut          │
│                                                                     │
│     c. updateVolume():                                              │
│        if target > smooth: α = ATTACK_SMOOTHING  (0.0046)           │
│        else:               α = RELEASE_SMOOTHING (0.0018)           │
│        smoothVolumeLinear += (tgtVol - smooth) * α                 │
│                                                                     │
│     d. sample(tone, phase, freq, smooth):                           │
│        one of 24 tone recipes (additive harmonics + FM + tanh)      │
│        phase += TWO_PI * freq / SAMPLE_RATE                         │
│                                                                     │
│     e. s = sample * smoothVolumeLinear * OUTPUT_GAIN * mixGain      │
│        (OUTPUT_GAIN = 0.14f, mixGain = 0.85f)                       │
│                                                                     │
│     f. if reverbEnabled:                                            │
│        delayed = combBuffer[combIdx]                                 │
│        combBuffer[combIdx] = clamp(s + delayed * 0.6f, -1, 1)       │
│        s = s*(1-reverbMix) + delayed*reverbMix                      │
│        combIdx = (combIdx + 1) & COMB_MASK  (8191)                  │
│                                                                     │
│     g. if delayEnabled:                                             │
│        delayed = delayBuffer[delayIdx]                              │
│        delayBuffer[delayIdx] = clamp(s + delayed*feedback, -1, 1)   │
│        s = s*(1-delayMix) + delayed*delayMix                        │
│        delayIdx = (delayIdx + 1) & DELAY_MASK  (32767)              │
│                                                                     │
│     h. if distortionEnabled:                                        │
│        s = tanh(s * gain) / tanh(gain)    (normalized soft-clip)    │
│                                                                     │
│     i. s = clamp(s, -1.0f, 1.0f)                                    │
│        monoBuffer[i] = (short)(s * 32767f)                          │
│                                                                     │
│  3. DrumEngine.mixInto(monoBuffer, 1024):                           │
│     for each active voice: monoBuffer[i] += pcm[pos] * vol *        │
│       globalDrumGain * DRUM_MIX_HEADROOM (0.26)                     │
│     voice pool: AtomicIntegerArray, up to MAX_VOICES=32             │
│                                                                     │
│  4. PcmListener.onPcmSamples(monoBuffer, 1024):                     │
│     → RecordingManager.onPcmSamples() if recording active          │
│       ReentrantLock.tryLock() — non-blocking; drops if finalizing   │
│                                                                     │
│  5. copyMonoToStereo():                                              │
│     stereoBuffer[2i]   = monoBuffer[i]  (left channel)              │
│     stereoBuffer[2i+1] = monoBuffer[i]  (right channel, L=R)        │
│                                                                     │
│  6. AudioTrack.write(stereoBuffer, 0, 2048):                        │
│     2048 shorts = 1024 frames × 2 channels                          │
│     Hardware driver queues for D/A conversion → Speaker             │
└─────────────────────────────────────────────────────────────────────┘

Design rationale and references. - The per-buffer pipeline is included because latency, jitter, and recording fidelity are ultimately determined at buffer granularity; this is the level where timing and side taps become concrete [R9][R11].

## 15. Function-Level Class Reference

All 34 Java source files in app/src/main/java/com/example/thereminglovestest2/.

Design rationale and references. - The function-level class reference is included for traceability so reviewers can verify that the architecture description matches the implemented codebase rather than an earlier design snapshot [R1].

#### ThereminAudioEngine

Real-time synthesis engine. Owns the AudioTrack, audio thread, effects pipeline, scale lock, vibrato, and PCM tap.

Key constants:

| Constant | Value | Purpose |
| --- | --- | --- |
| SAMPLE_RATE | 48000 | Hz; drives all timing math |
| AUDIO_WRITE_FRAMES | 1024 | Frames per AudioTrack write (~21.3 ms) |
| OUTPUT_GAIN | 0.14f | Global headroom guard (~−17 dB) |
| FREQ_SMOOTHING | 0.003f | IIR pitch smoothing coefficient (τ ≈ 6.9 ms) |
| ATTACK_SMOOTHING | 0.0046f | Faster volume envelope rise (τ ≈ 4.5 ms) |
| RELEASE_SMOOTHING | 0.0018f | Slower volume envelope decay (τ ≈ 11.6 ms) |
| VIBRATO_RATE_HZ | 4.2f | LFO frequency |
| MIN_VIBRATO_DEPTH | 0.0003f | Depth at near-silence |
| MAX_VIBRATO_DEPTH | 0.0014f | Depth at full volume |
| COMB_MASK | 8191 | Reverb buffer size − 1 (8192 samples = ~170 ms) |
| DELAY_MASK | 32767 | Delay buffer size − 1 (32768 samples = ~682 ms) |

Key methods:

| Method | Signature | Purpose |
| --- | --- | --- |
| start() | void | Clears effect buffers, primes smoothing, launches audio thread at THREAD_PRIORITY_AUDIO |
| stop() | void | Sets running=false, pauses/flushes/releases AudioTrack, joins thread with 500 ms timeout |
| setTargets() | (float freqHz, float volumeLinear) | Volatile write; audio thread reads on next buffer |
| fillBuffer() | private void fillBuffer(short[]) | Inner 1024-sample synthesis loop; hoists all volatile reads first |
| updateFrequency() | private float | IIR pitch smoothing + vibrato LFO computation |
| updateVolume() | private float | Asymmetric IIR (attack vs release) |
| sample() | private float sample(String, float, float, float) | Dispatches to one of 24 tone recipes |
| snapToScale() | private float snapToScale(float, String) | Binary search + 1-entry cache |
| applyReverb() | private float applyReverb(float, float) | Schroeder single-comb; bitwise AND wrap |
| applyDelay() | private float applyDelay(float, float, float) | Feedback delay line; bitwise AND wrap |
| applyDistortion() | private float applyDistortion(float, float) | tanh(x*g)/tanh(g) normalized soft clip |
| createAndStartTrack() | private AudioTrack | USAGE_GAME + CONTENT_TYPE_SONIFICATION, PERFORMANCE_MODE_LOW_LATENCY on API 26+ |

#### BleSessionManager

Process-wide static BLE host. Scans, connects, reconnects, runs the watchdog, stores cached MACs, parses telemetry, exposes BleSnapshot.

Key constants:

| Constant | Value |
| --- | --- |
| PITCH_NAME | "ThereminGlove" (right pitch glove) |
| VOLUME_NAME | "ThereminGloveVol" (left volume glove) |
| SCAN_TIMEOUT_MS | 12 000 ms |
| CONNECT_TIMEOUT_MS | 12 000 ms |
| AUTO_RECONNECT_DELAY_MS | 1 500 ms |
| PING_AFTER_MS | 3 000 ms |
| STALE_WARNING_MS | 4 500 ms |
| STALE_RECONNECT_MS | 20 000 ms |
| WATCHDOG_PERIOD_MS | 1 000 ms |

Key methods:

| Method | Purpose |
| --- | --- |
| initialize(Context) | Stores app context, restores cached MACs, starts watchdog |
| maybeStartAutoConnect() | Starts BLE scan if not already scanning and gloves missing |
| getSnapshot() | Returns a new immutable BleSnapshot from current static state |
| requestCaptureNeutral(boolean) | Sends N command to the right pitch glove or left volume glove |
| handleNotification(Glove, String) | Parses ACTIVE_DELTA_DEG, NEUTRAL_ROLL_DEG, DIRECTION packets |
| refreshTruth() | Watchdog body: checks timeouts, pings, forces reconnects |
| hasRequiredPermissions(Context) | API 31+: BLUETOOTH_SCAN+BLUETOOTH_CONNECT; API <31: ACCESS_FINE_LOCATION |

#### DrumEngine

PCM drum/bass/piano engine. Precomputes all sound data at construction using a hybrid bank of raw sample loads plus in-code synthesis; mixes directly into the shared theremin PCM path without SoundPool.

Key constants:

| Constant | Value |
| --- | --- |
| MAX_VOICES | 32 (concurrent PCM voices) |
| NUM_SOUNDS | 14 drum sounds |
| DRUM_MIX_HEADROOM | 0.26f |
| PIANO_MIX_HEADROOM | 0.22f |
| SAMPLE_RATE | 48 000 Hz |

Key methods:

| Method | Purpose |
| --- | --- |
| mixInto(short[], int) | Called from audio thread per buffer; mixes active drum/bass/piano voices |
| triggerSound(int) | Allocates a voice from the pool and fires a drum sound |
| tick() | Called by SequencerClock; advances step, fires sounds for current step |
| setCustomPattern(boolean[][]) | Replaces the active drum/bass step grid |
| clearCustomPattern() | Deactivates custom pattern; returns to preset |
| setBpm(int) | Updates sequencer tempo; takes effect on next tick |
| setEnabled(boolean) | Gates drum output on/off |
| setDrumGain(float) | Sets overall drum bus gain [0, 2] |

#### PlayMappingState

Pure mapping class with no Android framework dependencies. Converts live glove angles into frequency/volume targets.

Key methods:

| Method | Purpose |
| --- | --- |
| syncLive(BleSnapshot) | Copies live glove angles into instance fields |
| recompute(BleSnapshot) | Normalizes angles, applies sensitivity curve, octave shift, readiness check |
| normalizeClamped(float, float, float) | clamp((x−min)/(max−min), 0, 1) with zero-width guard |
| applySensitivityCurve(float) | pow(norm, sensitivityResponseCurve) |
| isInstrumentReady(BleSnapshot) | Returns false if Bluetooth off or either glove disconnected |

#### ThereminBackgroundAudioService

Android foreground service (mediaPlayback, START_NOT_STICKY). Keeps audio alive when MainActivity is not visible.

Static API:

| Method | Purpose |
| --- | --- |
| startIfNeeded(Context) | Starts the service if not already running |
| stopIfRunning(Context) | Stops and releases service engine |
| beginCalibrationPreview(Context, AppSettings) | Installs a temporary settings override for calibration preview |
| endCalibrationPreview() | Clears the preview override |
| setToneTypeNow(String) | Immediately sets tone on service engine (volatile write) |
| setActiveScale(String) | Sets scale lock (volatile write) |
| setReverbEnabled(boolean) | Toggles reverb in service engine |
| setOctaveShift(int) | Applied in sync loop before frequency computation |
| setSensitivityResponseCurve(float) | Updates sensitivity curve in sync loop |

#### RecordingManager

Implements ThereminAudioEngine.PcmListener. Writes WAV or AAC from the audio thread PCM tap.

Key constants:

| Constant | Value |
| --- | --- |
| MIN_FREE_BYTES | 10 MB (storage guard) |
| Sample rate | 48 000 Hz |
| Channels | 2 (stereo via mono duplication) |

Key methods:

| Method | Purpose |
| --- | --- |
| startRecording(Context, quality) | Opens WAV file or initializes MediaCodec+MediaMuxer |
| stopRecording() | Finalizes WAV header or calls MediaMuxer.stop(); acquires codecLock |
| onPcmSamples(short[], int) | Audio-thread tap; tryLock() prevents stall if finalizing |
| writePcmToWav(short[], int) | Writes mono buffer as stereo interleaved to RandomAccessFile |
| feedEncoder(short[], int) | Converts mono short[] to stereo float[]; feeds MediaCodec input buffer |

#### SettingsStore

SQLiteOpenHelper for theremin_gloves.db. Contains AppSettings as a static inner class.

Key methods:

| Method | Purpose |
| --- | --- |
| load(Context) | Opens DB, ensures schema, reads app_settings row 1 |
| save(Context, AppSettings) | Upserts app_settings row 1 |
| addColumnIfMissing(db, table, col, type, default) | Additive schema migration; never drops/renames |
| isCalibrationGuideLearned(Context) | Reads calibration_ui_prefs SharedPreferences flag |
| isBackgroundAudioEnabled(Context) | Reads theremin_prefs SharedPreferences flag |

#### RecordingRepository

SQLiteOpenHelper for recordings.db. Manages folders and recording metadata.

Tables: folders(id, name, created_at_ms) and recordings(id, file_path, display_name, duration_ms, created_at_ms, folder_id, quality).

Key methods: saveRecording(), getAllRecordings(), renameRecording(), deleteRecording() (DB + disk), createFolder(), getAllFolders() (with recording count via LEFT JOIN).

#### MainActivity (2778 lines)

Play screen. Owns foreground ThereminAudioEngine, DrumEngine, RecordingManager, PlayMappingState. Handles foreground/service audio ownership handoff.

Key fields:

| Field | Purpose |
| --- | --- |
| UI_TICK_MS = 50 | UI poll rate in milliseconds |
| TONE_CYCLE | Clone of AppSettings.USER_SELECTABLE_TONES (11 tones) |
| audioEngine | Foreground ThereminAudioEngine |
| drumEngine | Foreground DrumEngine (taken from AppLaunchWarmup) |
| playMapping | PlayMappingState for foreground path |
| recordingManager | Active RecordingManager |

Key methods: startAudio(), stopAudio(), onRecordButtonPressed(), refreshUiFast(), foreground/service handoff in onResume()/onPause().

#### CalibrationActivity (674 lines)

Calibration screen. Edits CalibrationDraft without touching saved settings until Save is pressed. Uses calibration preview path for live audio feedback.

Key methods: handleNeutralCapture(boolean) → BleSessionManager.requestCaptureNeutral(), syncCalibrationPreview() → ThereminBackgroundAudioService.beginCalibrationPreview(), saveAndPlay() → draft.saveTo(settings) → SettingsStore.save().

#### LibraryActivity (1414 lines)

Recording browser. Full CRUD on recordings and folders via RecordingRepository. Single MediaPlayer instance; mini-player; search; drag-to-folder; multi-select.

#### BeatMakerActivity (611 lines)

16-step sequencer editor. StepGridView 13×16 touch grid, PianoKeyboardView 25-key melody, local AudioTrack preview. Writes to DrumEngine on save.

#### AppLaunchWarmup

Startup accelerator. Background daemon thread pre-constructs DrumEngine, SettingsStore, AppSettings, RecordingRepository before MainActivity opens. take*() methods claim objects one-time.

#### BleSnapshot

Immutable value object returned by BleSessionManager.getSnapshot(). All fields final. Key fields: hostReady, pitchConnected, volumeConnected, pitchConnecting, volumeConnecting, pitchTelemetryStale, volumeTelemetryStale, pitchActiveDeltaDeg, volumeActiveDeltaDeg.

#### CalibrationDraft

Mutable staging buffer for unsaved calibration edits. sanitize() enforces valid min < max relationships. saveTo(AppSettings) finalizes to saved settings.

#### SequencerClock

Self-rescheduling ScheduledExecutorService 16th-note clock. Drift-free: each tick computes next delay from System.currentTimeMillis() delta, not a fixed interval.

#### PlayUiText

Stateless text formatter. Produces note-name strings (e.g., A4 / 440.0 Hz), volume display strings, and stage-mode pill labels for the Play screen UI.

#### RecordingExportManager

Creates user-visible export copies in Music/Theremin Gloves Recordings. The app-private original in getFilesDir()/recordings/ is always the canonical file.

#### RecordingListAdapter

RecyclerView.Adapter with two item types: TYPE_FOLDER and TYPE_RECORDING. Supports drag-into-folder gesture, multi-select, swipe-to-delete.

#### NavigationUtils

Static screen-transition helpers. Inner class Poller wraps a Handler.postDelayed loop with start()/stop() lifecycle helpers; used by all Activities for periodic UI refresh.

#### ThereminVisualizerView

Custom View. Renders 180-sample downsampled PCM waveform from ThereminAudioEngine.getVisualizerSnapshot(). Thread-safe: engine copies under visualizerLock.

#### ToneKnobView

Custom rotary dial. setToneSequence(String[]) with 11 tones. Rotary gesture cycles through USER_SELECTABLE_TONES.

#### BottomNavBarView

Custom View with five tabs: Play (0), Connect (1), Cal (2), Library (3), Settings (4). setTabGlowing(int, boolean) applies alpha-pulse animation to any tab for the onboarding hint system.

#### TopNavBarView

Custom View with back button, left action slot, two glove-status icon slots, and optional overflow menu.

#### StepGridView

Custom View. 13 rows × 16 columns touch grid. Rows 0–5: drum sounds. Rows 6–9: bass notes. Rows 10–12: extended sounds. Touch to toggle step on/off.

#### PianoKeyboardView

On-screen piano keyboard, MIDI C3–C5 (25 keys). Touch triggers DrumEngine.triggerPianoKey(). Used in BeatMaker.

#### PianoStepStripView

Horizontal strip showing which piano keys are scheduled in the current sequencer step column. Companion to PianoKeyboardView.

#### KnobControlView

Custom dial for numeric range adjustment. Reused for calibration angle/frequency tuning in CalibrationActivity.

#### InsetAwareScrollView

NestedScrollView subclass. Applies WindowInsetsCompat bottom padding automatically so content is not obscured by the navigation bar.

#### Instrument and SynthInstrument

Lightweight PCM abstractions. SynthInstrument wraps a volatile float[] PCM array. All fields volatile — safe for audio-thread reads while UI updates the reference.

#### HomeActivity

Setup/onboarding screen. Triggers BleSessionManager.maybeStartAutoConnect(). Auto-opens MainActivity when both gloves connect.

#### ConnectGlovesActivity

Manual BLE control screen. Per-glove connect/disconnect/reconnect buttons. Displays BleSnapshot.connectionDetail() text. Auto-navigates to Play when both gloves connected (unless EXTRA_SUPPRESS_AUTO_PLAY_REDIRECT extra is set).

#### LaunchActivity

App entry point. Calls AppLaunchWarmup.begin() and BleSessionManager.initialize(). Resolves Bluetooth permissions and Bluetooth-on state before routing to HomeActivity (first run) or MainActivity (returning user).

#### SettingsActivity

Exposes toggles for: background audio, extended frequency range, pitch/volume direction inversion, rename-on-stop dialog, recording quality, calibration guide, and sensitivity preset.

#### UserManualActivity

In-app user manual. Content is hardcoded in Java (no file reads). Mirrors docs/03_User_Manual.md.

## 16. Database Schema Diagram

theremin_gloves.db
└── app_settings  (single row, id = 1)
    ├── id                      INTEGER PRIMARY KEY
    ├── pitch_angle_min_deg     REAL    DEFAULT 0.0
    ├── pitch_angle_max_deg     REAL    DEFAULT 90.0
    ├── freq_min_hz             REAL    DEFAULT 20.0
    ├── freq_max_hz             REAL    DEFAULT 2000.0
    ├── volume_angle_min_deg    REAL    DEFAULT 0.0
    ├── volume_angle_max_deg    REAL    DEFAULT 90.0
    ├── pitch_direction_inverted INTEGER DEFAULT 0
    ├── volume_direction_inverted INTEGER DEFAULT 1
    ├── tone_type               TEXT    DEFAULT 'SINE' → normalised to THEREMIN
    ├── pitch_enabled           INTEGER DEFAULT 1
    ├── volume_enabled          INTEGER DEFAULT 1
    ├── updated_at_ms           INTEGER DEFAULT 0
    ├── active_scale            TEXT    DEFAULT 'CHROMATIC'
    ├── octave_shift            INTEGER DEFAULT 0
    ├── reverb_enabled          INTEGER DEFAULT 0
    ├── reverb_mix              REAL    DEFAULT 0.3
    ├── delay_enabled           INTEGER DEFAULT 0
    ├── delay_feedback          REAL    DEFAULT 0.35
    ├── delay_mix               REAL    DEFAULT 0.4
    ├── distortion_enabled      INTEGER DEFAULT 0
    ├── distortion_gain         REAL    DEFAULT 3.0
    ├── sensitivity_level       TEXT    DEFAULT 'MEDIUM'
    └── sensitivity_curve       REAL    DEFAULT 1.0

recordings.db
├── folders
│   ├── id              INTEGER PRIMARY KEY AUTOINCREMENT
│   ├── name            TEXT NOT NULL
│   └── created_at_ms   INTEGER NOT NULL
│
└── recordings
    ├── id              INTEGER PRIMARY KEY AUTOINCREMENT
    ├── file_path       TEXT NOT NULL
    ├── display_name    TEXT NOT NULL
    ├── duration_ms     INTEGER NOT NULL
    ├── created_at_ms   INTEGER NOT NULL
    ├── folder_id       INTEGER REFERENCES folders(id)  (NULL = root)
    └── quality         TEXT NOT NULL  ('LOSSLESS', 'HIGH', 'MEDIUM', 'LOW')

Migration strategy: SettingsStore.addColumnIfMissing(db, table, col, type, default) is called in onOpen() for every column that was added after version 1. Columns are never dropped or renamed. The schemaVerifiedForProcess flag prevents re-running schema checks on every screen load within one app session.

Design rationale and references. - The schema diagram is included because persistence is part of the system architecture, not just a storage detail. This is especially important here because the project uses a lightweight raw-SQLite layer rather than Room-generated schema metadata [R1][R15][R16].

## 17. UML Diagrams

Design rationale and references. - The UML views are included because they provide standardized static, behavioral, and component-level perspectives that complement the prose architecture description [R1].

### 17.1 UML Class Diagram

Core classes, representative members, and inter-class relationships. Member names and dependencies are source-verified against the sprint3 codebase.

classDiagram
    class ThereminAudioEngine {
        +SAMPLE_RATE : int
        +AUDIO_WRITE_FRAMES : int
        +OUTPUT_GAIN : float
        -targetFreqHz : float
        -targetVolumeLinear : float
        -toneType : String
        -activeScale : String
        -drumEngine : DrumEngine
        -pcmListener : PcmListener
        +start()
        +stop()
        +setTargets(freqHz, volumeLinear)
        +setToneType(tone)
        +setDrumEngine(engine)
        +setPcmListener(listener)
        +getVisualizerSnapshot()
        -fillBuffer()
        -sample(tone, phase, freq)
        -snapToScale(freq)
        -applyReverb(sample)
        -applyDelay(sample)
        -applyDistortion(sample)
    }

    class ThereminBackgroundAudioService {
        +SYNC_TICK_MS : int
        +SETTINGS_REFRESH_MS : int
        -engine : ThereminAudioEngine
        -drumEngine : DrumEngine
        -mappingState : PlayMappingState
        +onStartCommand(intent, flags, startId)
        +onDestroy()
        -syncLoop()
        -pushTargets()
    }

    class BleSessionManager {
        +SCAN_TIMEOUT_MS : long
        +AUTO_RECONNECT_DELAY_MS : long
        +PING_AFTER_MS : long
        +STALE_WARNING_MS : long
        +STALE_RECONNECT_MS : long
        -pitchGlove : Glove
        -volumeGlove : Glove
        +getSnapshot() BleSnapshot
        +requestScan(context)
        +requestDisconnect()
        -watchdog()
    }

    class BleSnapshot {
        +pitchDeltaDeg : float
        +volumeDeltaDeg : float
        +isPitchConnected : boolean
        +isVolumeConnected : boolean
        +pitchNeutralDeg : float
        +volumeNeutralDeg : float
    }

    class PlayMappingState {
        -pitchAngleMin : float
        -pitchAngleMax : float
        -freqMinHz : float
        -freqMaxHz : float
        -sensitivityResponseCurve : float
        -octaveShift : int
        +recompute(BleSnapshot snapshot)
        +getFrequencyHz() float
        +getVolumeLinear() float
        -normalizeClamped(value, low, high)
    }

    class DrumEngine {
        +MAX_VOICES : int
        +NUM_SOUNDS : int
        +DRUM_MIX_HEADROOM : float
        +PIANO_MIX_HEADROOM : float
        -clock : SequencerClock
        -sounds : float[][]
        +mixInto(buffer, frames)
        +setBpm(bpm)
        +setPattern(track, step, enabled)
        +start()
        +stop()
    }

    class SequencerClock {
        -lastTickMs : long
        +schedule(tick, intervalMs)
        +stop()
    }

    class RecordingManager {
        -codecLock : ReentrantLock
        -activeQuality : Quality
        +start(quality)
        +stop()
        +onPcmSamples(buffer, length)
    }

    class SettingsStore {
        -DB_NAME : String
        +save(settings)
        +load() AppSettings
        -ensureSchema(database)
        -addColumnIfMissing(database, column, definition)
    }

    class AppSettings {
        +pitchAngleMinDeg : float
        +pitchAngleMaxDeg : float
        +freqMinHz : float
        +freqMaxHz : float
        +toneType : String
        +activeScale : String
        +octaveShift : int
        +reverbEnabled : boolean
        +reverbMix : float
        +sensitivityResponseCurve : float
    }

    class RecordingRepository {
        -DB_NAME : String
        +insert(recording)
        +getAll()
        +getByFolder(folderId)
        +delete(recordingId)
        +updateDisplayName(recordingId, name)
    }

    class AppLaunchWarmup {
        -daemonThread : Thread
        +start(context)
        +takeDrumEngine() DrumEngine
        +takeSettingsStore() SettingsStore
        +takeRecordingRepository() RecordingRepository
    }

    class PcmListener {
        +onPcmSamples(buffer, length)
    }

    class CalibrationDraft {
        +pitchAngleMin : float
        +pitchAngleMax : float
        +freqMinHz : float
        +freqMaxHz : float
        +sanitize()
        +saveTo(target)
        +applyFrom(source)
    }

    ThereminBackgroundAudioService --> ThereminAudioEngine : owns
    ThereminBackgroundAudioService --> DrumEngine : owns
    ThereminBackgroundAudioService --> PlayMappingState : owns
    ThereminBackgroundAudioService ..> SettingsStore : reads settings

    BleSessionManager ..> BleSnapshot : creates snapshot

    ThereminAudioEngine --> DrumEngine : mixes drum audio
    ThereminAudioEngine --> PcmListener : emits PCM tap

    RecordingManager ..|> PcmListener : implements

    PlayMappingState ..> BleSnapshot : reads live angles

    SettingsStore --> AppSettings : loads and saves
    CalibrationDraft --> AppSettings : commits values

    DrumEngine --> SequencerClock : uses

    AppLaunchWarmup ..> DrumEngine : preconstructs
    AppLaunchWarmup ..> SettingsStore : preconstructs
    AppLaunchWarmup ..> RecordingRepository : preconstructs

Design rationale and references. - The class diagram exists to expose the main static relationships between BLE, mapping, synthesis, persistence, and warm-up infrastructure in a standardized visual form [R1].

### 17.2 UML Sequence Diagram — BLE Packet to Audio Output

End-to-end signal path from glove firmware through BLE, mapping, synthesis, effects, drum mix, and final AudioTrack write. All timing constants are source-verified.

sequenceDiagram
    participant G as GloveFirmware
    participant B as BleSessionManager
    participant H as UITickHandler
    participant P as PlayMappingState
    participant E as ThereminAudioEngine
    participant D as DrumEngine
    participant R as RecordingManager
    participant A as AudioTrack

    G->>B: BLE notify "ACTIVE_DELTA_DEG:30.5"
    B->>B: parse ASCII packet
    B->>B: update Glove.activeDeltaDeg

    Note over H: every UI_TICK_MS = 50 ms
    H->>B: getSnapshot()
    B-->>H: BleSnapshot (immutable copy)
    H->>P: recompute(snapshot)
    P->>P: normalizeClamped(angle, min, max)
    P->>P: pow(norm, sensitivityCurve)
    P->>P: freqHz = freqMin + span × curved
    P->>P: freqHz × 2^octaveShift
    P-->>H: freqHz=1600, volumeLinear=0.80
    H->>E: setTargets(1600.0f, 0.80f)

    Note over E: every ~21.3 ms (THREAD_PRIORITY_AUDIO)
    E->>E: fillBuffer() — hoist all volatiles to finals
    E->>E: IIR freq smooth: α=FREQ_SMOOTHING=0.003
    E->>E: snapToScale(): binary search prebuilt MIDI table
    E->>E: vibrato: freq × (1 + sin(phase) × depth)
    E->>E: IIR vol smooth: α=ATTACK or RELEASE (0.0046 / 0.0018)
    E->>E: sample(toneType): additive synthesis (24 recipes)
    E->>E: × OUTPUT_GAIN=0.14 × mixGain
    E->>E: applyReverb(): Schroeder comb, g=0.6, L=8192
    E->>E: applyDelay(): ring buffer, L=32768
    E->>E: applyDistortion(): tanh(x·gain)/tanh(gain)
    E->>D: mixInto(monoBuffer, 1024)
    D-->>E: drum samples added (DRUM_MIX_HEADROOM=0.26)
    E->>R: onPcmSamples(monoBuffer, 1024)
    R->>R: tryLock() — encode WAV/AAC if active
    E->>E: copyMonoToStereo → stereoBuffer[2048]
    E->>A: write(stereoBuffer, 0, 2048, WRITE_BLOCKING)
    A-->>Speaker: audio output

Design rationale and references. - The sequence diagram is included because the core claim of the product is real-time gesture-to-sound translation; a sequence view makes causality and timing boundaries explicit [R1][R11].

### 17.3 UML Activity Diagram — Calibration Flow

User journey from cold start through calibration to live play. The CalibrationDraft buffers all edits; saveTo(AppSettings) commits to SQLite only on explicit save.

flowchart TD
    A([App Launch]) --> B{Permissions\ngranted?}
    B -- No --> C[Request BLUETOOTH_SCAN\nBLUETOOTH_CONNECT\nNEARBY_DEVICES]
    C --> B
    B -- Yes --> D[LaunchActivity:\nWarmup pre-constructs\nDrumEngine + SettingsStore]
    D --> E[HomeActivity:\nAuto-scan for saved MAC addresses]
    E --> F{Both gloves\nconnected?}
    F -- No --> G[ConnectGlovesActivity:\nManual scan / connect]
    G --> F
    F -- Yes --> H[CalibrationActivity opens]
    H --> I[PITCH tab:\nPress 'Pitch Neutral'\nto capture neutral roll angle]
    I --> J[Glove sends 'N' command;\nGlove firmware saves neutral position]
    J --> K[Switch to VOLUME tab:\nPress 'Volume Neutral']
    K --> L[Glove sends 'N' command]
    L --> M[Adjust angle ranges\nfreq range, direction inversion]
    M --> N{Preview\nrequested?}
    N -- Yes --> O[beginCalibrationPreview:\nBackground service uses\nCalibrationDraft values live]
    O --> M
    N -- No --> P[Press 'Save & Play']
    P --> Q[CalibrationDraft.sanitize:\nclamp zero-width ranges]
    Q --> R[saveTo AppSettings:\nSettingsStore.save to SQLite]
    R --> S[MainActivity: Play screen\nForeground ThereminAudioEngine starts]
    S --> T([Live theremin playing])

Design rationale and references. - The activity diagram is included because calibration is one of the most stateful user-facing workflows in the app and includes preview, device-side neutral capture, validation, and persistence [R1][R12].

### 17.4 UML Component Diagram

Physical and logical component boundaries from hardware to user interface.

flowchart TB
    subgraph Hardware["Hardware Layer"]
        GL1["Right Pitch Glove\nArduino Nano 33 BLE Sense\nLSM9DS1 IMU"]
        GL2["Left Volume Glove\nArduino Nano 33 BLE Sense\nLSM9DS1 IMU"]
    end

    subgraph BLE["BLE Transport"]
        PKT["Packet: ACTIVE_DELTA_DEG:&lt;f&gt;\nNEUTRAL_ROLL_DEG:&lt;f&gt;\nDIRECTION:&lt;POS|NEG&gt;"]
        CMD["Commands: H ping / N neutral / D direction"]
    end

    subgraph Android["Android Process"]
        subgraph BLEMgr["BleSessionManager (singleton)"]
            SCAN["Scanner"]
            GATT["GATT client (Nordic BLE 2.11)"]
            WD["Watchdog 1 s tick"]
            SNAP["BleSnapshot (immutable)"]
        end

        subgraph Mapping["Mapping Layer"]
            PMS["PlayMappingState\nangle → freq / vol"]
        end

        subgraph FG["Foreground Path (MainActivity)"]
            TAE1["ThereminAudioEngine\n48 kHz / 1024-frame buffer"]
            DE1["DrumEngine\n14 sounds / 8 patterns"]
            RM["RecordingManager\nPcmListener"]
        end

        subgraph SVC["Background Path (ThereminBackgroundAudioService)"]
            TAE2["ThereminAudioEngine\n(service-owned copy)"]
            DE2["DrumEngine\n(service-owned copy)"]
        end

        subgraph Persist["Persistence"]
            SS["SettingsStore\ntheremin_gloves.db\n24 columns"]
            RR["RecordingRepository\nrecordings.db"]
        end

        subgraph UI["UI Layer"]
            MA["MainActivity\n(Play screen)"]
            LA["LibraryActivity"]
            CA["CalibrationActivity"]
            BA["BeatMakerActivity"]
        end
    end

    GL1 -- "BLE GATT notify" --> PKT
    GL2 -- "BLE GATT notify" --> PKT
    PKT --> GATT
    CMD --> GL1
    CMD --> GL2
    GATT --> SCAN
    SCAN --> WD
    GATT --> SNAP
    SNAP --> PMS
    PMS --> TAE1
    PMS --> TAE2
    TAE1 --> DE1
    TAE1 --> RM
    TAE2 --> DE2
    RM --> RR
    SS --> MA
    SS --> SVC
    MA --> TAE1
    LA --> RR
    CA --> SS
    BA --> DE1

Design rationale and references. - The component diagram is included because the project spans hardware, firmware, BLE transport, Android services, UI, and storage; a component view makes deployment-scale relationships clearer than a class view alone [R1].

## References

[R1] ISO/IEC/IEEE 42010 architecture overview. https://www.iso-architecture.org/ieee-1471/ads/
[R2] Arduino Nano 33 BLE Sense Rev2 documentation. https://docs.arduino.cc/hardware/nano-33-ble-sense-rev2/
[R3] Android Developers, BLE overview. https://developer.android.com/develop/connectivity/bluetooth/ble/ble-overview
[R4] Android Developers, communicate with BLE devices in the background. https://developer.android.com/develop/connectivity/bluetooth/ble/background
[R5] Android Developers, Bluetooth permissions. https://developer.android.com/develop/connectivity/bluetooth/bt-permissions
[R6] Nordic Semiconductor, Android BLE Library. https://github.com/nordicsemi/Android-BLE-Library
[R7] Android Developers, audio latency guidance. https://developer.android.com/ndk/guides/audio/audio-latency
[R8] Android Developers, low latency audio on Android / Oboe guidance. https://developer.android.com/games/sdk/oboe/low-latency-audio
[R9] Android Developers, AudioTrack, AudioAttributes, and background playback guidance. https://developer.android.com/reference/android/media/AudioTrack ; https://developer.android.com/reference/android/media/AudioAttributes ; https://developer.android.com/media/media3/session/background-playback
[R10] Wessel, D., and Wright, M. “Problems and Prospects for Intimate Musical Control of Computers,” 2002. https://cnmat.berkeley.edu/publications/problems-and-prospects-intimate-musical-control-computers-0
[R11] McPherson, A., Jack, R., and Moro, G. “Action-Sound Latency: Are Our Tools Fast Enough?” NIME 2016. https://nime.org/proc/mcpherson2016/
[R12] Review of user-conducted IMU calibration methods. https://www.sciencedirect.com/science/article/abs/pii/S0263224123015658
[R13] Mäki-Patola, T., and Hämäläinen, P. latency tolerance for gesture-controlled continuous sound instruments. https://users.aalto.fi/~hamalap5/publications/icmcarticlefinal10.pdf
[R14] Android Developers, supported media formats. https://developer.android.com/media/platform/supported-formats
[R15] Android Developers, Room persistence guidance. https://developer.android.com/training/data-storage/room/
[R16] SQLite ALTER TABLE guidance. https://www.sqlite.org/lang_altertable.html
[R17] Android Developers, SharedPreferences guidance. https://developer.android.com/training/data-storage/shared-preferences
[R18] Android Developers, adaptive orientation and resizability guidance. https://developer.android.com/develop/ui/compose/layouts/adaptive/app-orientation-aspect-ratio-resizability
[R19] Clarinet acoustics, University of New South Wales. https://www.phys.unsw.edu.au/jw/clarinetacoustics.html
[R20] Flute acoustics, University of New South Wales. https://www.phys.unsw.edu.au/jw/fluteacoustics.html
[R21] Sawtooth-wave Fourier series. https://mathworld.wolfram.com/FourierSeriesSawtoothWave.html
[R22] Square-wave Fourier series. https://mathworld.wolfram.com/FourierSeriesSquareWave.html
[R23] Triangle-wave Fourier series. https://mathworld.wolfram.com/FourierSeriesTriangleWave.html
[R24] Typical vibrato-rate review. https://pmc.ncbi.nlm.nih.gov/articles/PMC9510150/
[R25] Android Developers, MediaPlayer reference. https://developer.android.com/reference/android/media/MediaPlayer